SMART PARKING MANAGEMENT SYSTEM

A Distributed Embedded Architecture with Interrupt-Driven NFC,
Real-Time FreeRTOS Scheduling, and Machine Learning-Assisted
License Plate Recognition

Final Project Report

Embedded Systems — IoT Smart Device

Faculty of Electrical and Electronics Engineering

2026

# CHAPTER 1: INTRODUCTION AND PROBLEM STATEMENT

## 1.1 Background and Motivation

Urban parking management has emerged as one of the most persistent logistical challenges in densely populated cities, university campuses, commercial complexes, and residential zones across the world. As vehicle ownership continues to grow at rates that far outpace the expansion of physical parking infrastructure, the inefficiencies of traditional manual parking systems become increasingly costly — both economically and operationally.

Conventional parking management in small-to-medium facilities typically relies on human attendants who manually record vehicle entries and exits using paper-based logbooks or, at best, rudimentary spreadsheets. This approach introduces a cascade of systemic problems: clerical errors, slow throughput during peak hours, the absence of a reliable audit trail, susceptibility to fraud, and an inability to monitor real-time occupancy. In contexts such as university campuses or office buildings, where hundreds of vehicles cycle through each day, these limitations translate into quantifiable economic losses and measurable user dissatisfaction.

The advent of low-cost embedded microcontrollers with integrated wireless connectivity — exemplified by the Espressif ESP32 family — has dramatically lowered the barrier to deploying intelligent, connected physical systems. When combined with mature real-time operating system frameworks such as FreeRTOS, cloud-capable communication protocols such as MQTT and HTTP, and emerging machine learning services for computer vision, these microcontrollers enable the construction of sophisticated, autonomous embedded systems that were previously achievable only with expensive dedicated hardware.

This project leverages precisely this confluence of technologies to design, implement, and evaluate a fully functional Smart Parking Management System. The system replaces manual parking operations with an automated, distributed embedded architecture that employs NFC-based vehicle identification, camera-based snapshot evidence capture, machine learning-driven license plate recognition, and a real-time web dashboard — all coordinated through a carefully designed FreeRTOS task model and MQTT message broker.

## 1.2 Problem Statement

The specific operational context motivating this project is a small-to-medium parking facility (20–100 vehicle capacity) such as those commonly found at university campuses or office complexes in Vietnam. Based on direct observation and stakeholder interviews, the following concrete problems were identified:

Manual vehicle check-in and check-out introduce human error and processing delays of 15–30 seconds per vehicle, causing queues during peak entry and exit times.

No photographic evidence is captured at the time of entry, making it impossible to resolve disputes about vehicle identity, damage, or unauthorized access after the fact.

There is no real-time mechanism for parking staff to monitor how many vehicles are currently in the facility, which slots are occupied, or which cards are currently active.

Barrier gates are operated manually, requiring a staff member to physically interact with a control panel — a bottleneck that compounds during high-traffic periods.

There is no centralized log of vehicle entry and exit events, making retrospective auditing for security incidents impossible.

The system cannot scale: adding a second or third gate requires adding personnel, not just hardware.

These problems collectively define the technical challenge that this project addresses. The solution must be fast enough to eliminate queue formation, reliable enough to operate continuously without human supervision, cost-effective enough to deploy in resource-constrained environments, and architecturally sound enough to scale across multiple lanes with minimal modification.

## 1.3 Proposed Solution Overview

The proposed Smart Parking Management System replaces the manual workflow with a distributed embedded architecture consisting of three classes of physical nodes, all communicating over a standard IEEE 802.11 (WiFi) local area network:

NFC and Barrier Control Node (ESP32 WROOM32): A microcontroller node equipped with a PN532 NFC reader module and two SG90 servo motors controlling entry and exit barrier gates. This node acts as the local master of the parking state machine — it maintains an in-memory table of currently parked vehicles, makes autonomous entry/exit decisions based on card UID lookups, and physically actuates barriers in response to License Plate Recognition (LPR) results received over MQTT.

Camera Nodes — Entry and Exit (ESP32-CAM): Two identical ESP32-CAM modules, each equipped with an OV2640 image sensor and 4 MB on-board PSRAM. Each camera node subscribes to MQTT trigger commands from the NFC node, captures a JPEG snapshot upon trigger, streams a live MJPEG video feed, runs the LPR pipeline (via a server-proxied cloud API call), and publishes the recognition result back over MQTT.

Controller Node (Laptop/PC running FastAPI): A Python-based backend server that acts as the system's observer and administrative interface. It receives structured log data from the camera nodes via HTTP, maintains a session store of active parking records, serves a real-time web dashboard via Server-Sent Events (SSE), proxies the MJPEG video streams, and allows administrators to override barrier gates manually via an authenticated web interface.

The three nodes communicate primarily through an MQTT message broker, with HTTP/REST used for log uploads from camera nodes to the backend server, and SSE used for pushing real-time events from the server to the web dashboard. This architecture ensures that the critical path — NFC scan to barrier opening — operates entirely on embedded hardware without depending on the server, making the system resilient to server unavailability.

## 1.4 Objectives of the Project

The primary and secondary objectives of this project are enumerated below:

Primary Objective: Design and implement a fully automated parking management system that replaces manual vehicle check-in and check-out with an NFC-triggered, camera-verified, barrier-controlled workflow.

Embedded Systems Objective: Demonstrate the application of advanced embedded systems techniques including interrupt-driven peripheral management, hardware and software timer usage, FreeRTOS multi-task scheduling with proper synchronization primitives, DMA-based high-speed data transfer, and PSRAM dynamic memory management — all within the constraints of a resource-limited microcontroller environment.

Real-Time Performance Objective: Achieve a total latency from NFC card presentation to barrier opening of under 3 seconds under normal operating conditions, and a NFC event to dashboard notification latency of under 1 second.

Reliability Objective: Implement comprehensive error handling, hardware watchdog supervision, and automatic recovery mechanisms such that the system can operate continuously for extended periods without requiring human intervention or manual restart.

Scalability Objective: Design the system architecture such that adding additional entry or exit lanes requires only the deployment of additional ESP32-CAM nodes, with no changes to the NFC node or backend server code.

Documentation Objective: Produce a complete academic thesis that rigorously documents the design decisions, implementation details, performance measurements, and identified limitations of the system.

## 1.5 Scope and Limitations

The project is explicitly scoped to demonstrate technical feasibility at the prototype scale. The following boundaries define the scope:

Hardware: One NFC/barrier node, one entry ESP32-CAM, one exit ESP32-CAM, all connected to a single WiFi access point on a local area network.

Vehicle identification: Based exclusively on NFC card UID (ISO 14443A standard). RFID cards, not vehicle transponders, are used as the identification medium.

License plate recognition: Delegated to a third-party cloud API (Plate Recognizer) proxied through the backend server to offload TLS and processing overhead from the ESP32-CAM.

Session persistence: Session data is held in memory on the backend server. A server restart clears all active session records. Persistent storage (database) is identified as a future enhancement.

Security: No encryption of MQTT messages or HTTP API endpoints is implemented in the prototype. API key authentication for the barrier override endpoint is identified as a future enhancement.

Scale: The system is tested with a single parking lane pair (one entry, one exit). Multi-lane expansion is architecturally supported but not physically tested.

These limitations are acknowledged and discussed further in Chapter 6. They do not diminish the technical validity of the implemented embedded systems techniques, which would apply equally in a production-scale deployment.

## 1.6 Report Structure

The remainder of this report is organized as follows. Chapter 2 provides a literature review covering the theoretical foundations of the key technologies employed: FreeRTOS real-time task scheduling, the ESP32 microcontroller architecture, NFC communication standards, MQTT protocol semantics, and machine learning-based license plate recognition. Chapter 3 presents the complete system design, encompassing hardware selection and connectivity, the multi-node network architecture, the layered software model, and the finite state machine governing system behavior. Chapter 4 constitutes the core of the report, providing a detailed account of the implementation of every major subsystem, with annotated code excerpts and design rationale. Chapter 5 describes the testing methodology, presents performance measurement results, and analyzes the system's behavior against its stated requirements. Chapter 6 concludes the report with a summary of achievements, an honest assessment of limitations, and a roadmap for future enhancements.

# CHAPTER 2: LITERATURE REVIEW AND BACKGROUND THEORY

## 2.1 Embedded Systems versus IoT: A Critical Distinction

The terms 'embedded system' and 'IoT device' are frequently used interchangeably in popular discourse, yet they represent meaningfully different design philosophies with distinct engineering implications. Understanding this distinction is essential for evaluating the design choices made in this project.

An embedded system, in the classical sense, is a computing system designed to perform a specific, well-defined function within a larger mechanical or electrical system. The defining characteristics of embedded systems are real-time responsiveness, deterministic behavior, resource-constrained operation, tight hardware-software integration, and often continuous operation without human supervision. Classic examples include automotive engine control units, industrial programmable logic controllers, medical device firmware, and aerospace flight control systems.

An IoT (Internet of Things) device, by contrast, is typically characterized by its connectivity to a network — usually the internet — and its role as a data source or actuator in a broader cloud-centric ecosystem. Many entry-level IoT implementations follow a thin-client model: the device collects sensor data and transmits it to a cloud service, which performs all meaningful computation and returns commands. In such architectures, the embedded hardware is reduced to a glorified sensor adapter, and the system's intelligence resides entirely in the cloud.

The Smart Parking System presented in this report deliberately bridges both paradigms. The NFC and Barrier Control Node operates as a true embedded system: it maintains a local parking state machine, makes autonomous entry/exit decisions, manages hardware peripherals (I2C, PWM servo) in real time via interrupt-driven and timer-based mechanisms, and does not depend on server availability to perform its primary function. The camera nodes implement a sophisticated FreeRTOS multi-task firmware that manages DMA-based image capture, PSRAM memory allocation, and concurrent MJPEG streaming — all hallmarks of embedded systems engineering. The server's role is explicitly scoped to logging, monitoring, and administrative override — not to the critical path of vehicle processing.

## 2.2 FreeRTOS: Real-Time Operating System Fundamentals

FreeRTOS (Free Real-Time Operating System) is an open-source, POSIX-inspired real-time kernel originally developed by Richard Barry and now maintained by Amazon Web Services. It is one of the most widely deployed RTOSes in the embedded systems industry, supported on over 40 processor architectures and found in products ranging from industrial sensors to automotive components. The ESP32 SDK (ESP-IDF) includes FreeRTOS as its primary task management framework, making it the natural choice for this project.

FreeRTOS provides the following key primitives that are exploited in this project:

### 2.2.1 Tasks

A FreeRTOS task is a self-contained thread of execution with its own stack, priority level, and state. The FreeRTOS scheduler implements a preemptive, priority-based scheduling algorithm: at any tick, the highest-priority ready task runs. Tasks with equal priority share the CPU via round-robin time-slicing. The scheduler itself is driven by the SysTick hardware interrupt, which fires at a configurable rate (typically 1000 Hz on ESP32, giving 1 ms tick resolution). Task creation with xTaskCreatePinnedToCore() on the dual-core ESP32 allows explicit binding of a task to either Core 0 (PRO_CPU) or Core 1 (APP_CPU), enabling deterministic CPU isolation for time-critical workloads.

### 2.2.2 Queues

FreeRTOS queues provide a thread-safe, copy-based inter-task communication mechanism. A queue has a fixed item size and depth. The xQueueSend() family of functions copies an item into the queue, blocking the sender if the queue is full (with configurable timeout). xQueueReceive() copies an item out, blocking the receiver if the queue is empty. xQueueOverwrite() is a special variant for depth-1 queues that overwrites the existing item without blocking, guaranteeing that the most recent event is always available. This project uses queues as the primary pipeline between firmware stages: NFC event → camera capture → LPR → log upload.

### 2.2.3 Semaphores and Mutexes

Binary semaphores and mutexes are the canonical FreeRTOS synchronization primitives. A binary semaphore is a signaling mechanism — one task 'gives' it and another 'takes' it, effectively acting as a lightweight event notification. A mutex (mutual exclusion semaphore) additionally implements priority inheritance: if a high-priority task is blocked waiting for a mutex held by a low-priority task, the low-priority task's priority is temporarily elevated to prevent priority inversion. This project uses mutexes to protect the global FSM state variable and the camera DMA framebuffer, both of which are accessed from multiple concurrent tasks.

### 2.2.4 Software Timers

FreeRTOS software timers (xTimerCreate, xTimerStart) allow callback functions to be scheduled for execution after a specified delay or at a fixed period, without requiring a dedicated task to block. Timer callbacks execute in the context of a dedicated Timer Service Task (also called the RTOS Daemon Task). This project proposes using FreeRTOS software timers to implement the gate auto-close mechanism — a 5-second one-shot timer that closes the servo barrier after it has been opened, replacing the previous approach of blocking an entire task thread with vTaskDelay(5000ms).

### 2.2.5 Task Notifications

Introduced in FreeRTOS version 8.2, Task Notifications provide a lightweight alternative to binary semaphores for direct task-to-task signaling. Each task has a 32-bit notification value stored directly in its Task Control Block (TCB). xTaskNotifyGive() increments this value (equivalent to a binary semaphore give), and ulTaskNotifyTake() decrements and blocks (equivalent to a binary semaphore take). Because no separate kernel object is allocated, Task Notifications consume less RAM and execute faster than equivalent semaphore operations — an important consideration on memory-constrained microcontrollers.

## 2.3 ESP32 Microcontroller Architecture

The Espressif ESP32 is a dual-core 32-bit microcontroller based on the Xtensa LX6 processor architecture, operating at up to 240 MHz. It integrates 520 KB of on-chip SRAM, 448 KB of ROM, a hardware IEEE 802.11 b/g/n WiFi radio, a Bluetooth 4.2/BLE radio, and a rich set of peripherals including I2C, SPI, UART, I2S, PWM (LEDC), ADC, DAC, and touch sensor interfaces. The ESP32 is available in various module formats; this project uses the ESP32 WROOM32 module for the NFC/barrier node and the AI-Thinker ESP32-CAM module for the camera nodes.

### 2.3.1 Dual-Core Architecture

The two Xtensa LX6 cores in the ESP32 — designated Core 0 (PRO_CPU) and Core 1 (APP_CPU) — are symmetric in capability but asymmetric in their default usage by the ESP-IDF framework. The WiFi and Bluetooth protocol stacks are pinned to Core 0, making it responsible for radio interrupt handling, TCP/IP stack processing, and network I/O. Application tasks pinned to Core 0 therefore share CPU time with radio interrupts, while tasks pinned to Core 1 are free from this interference. This asymmetry has direct implications for latency-sensitive workloads: camera capture and MJPEG stream tasks are pinned to Core 1 to prevent WiFi interrupt preemption from corrupting DMA operations or causing frame drops.

### 2.3.2 PSRAM (Pseudo-Static RAM)

The ESP32-CAM module includes 4 MB of SPI-connected PSRAM (Pseudo-Static RAM), accessible via the ESP32's SPI interface at up to 80 MHz. PSRAM provides a large heap extension beyond the 520 KB of internal SRAM, which is insufficient to hold even a single uncompressed SVGA (800×600) image. The esp_camera library automatically configures camera framebuffers to reside in PSRAM when configured with fb_location = CAMERA_FB_IN_PSRAM. PSRAM is also used in this project for dynamic allocation of JPEG image buffers during LPR processing and HTTP multipart upload construction, via the heap_caps_malloc(size, MALLOC_CAP_SPIRAM) API.

### 2.3.3 I2S and DMA

The OV2640 camera sensor outputs pixel data over a parallel digital interface (DVP: Data, VSYNC, HREF, PCLK). The ESP32 receives this data stream using its I2S peripheral in LCD/Camera mode, which internally uses DMA (Direct Memory Access) channels to transfer pixel data from the I2S FIFO into PSRAM framebuffers without CPU involvement. This means that during a frame capture, the CPU is free to perform other work — a key advantage for maintaining concurrent MJPEG streaming. The DMA transfer is initiated and completed transparently within the esp_camera_fb_get() call; by the time the function returns, the entire JPEG-compressed frame is already in the PSRAM framebuffer.

### 2.3.4 LEDC (LED Control) Peripheral

The LEDC peripheral is a dedicated hardware PWM generator in the ESP32 that can produce up to 16 independent PWM channels at configurable frequencies and resolutions. Despite its name suggesting LED dimming, LEDC is fully general-purpose and is the standard mechanism for servo motor control in the Arduino-ESP32 ecosystem. LEDC channels are driven by internal hardware timers, meaning PWM signal generation is completely autonomous — the CPU does not need to toggle GPIO pins manually or service any timer interrupts to maintain the signal. This project uses LEDC to generate 50 Hz / 16-bit PWM signals on two GPIO pins for the entry and exit SG90 servo motors.

## 2.4 NFC Technology and ISO 14443A Standard

Near Field Communication (NFC) is a set of short-range wireless communication protocols operating at 13.56 MHz, standardized by ISO/IEC 18000-3. For contactless smart card identification — the use case in this project — the relevant standard is ISO 14443A, which defines the physical layer, anti-collision protocol, and data link layer for proximity cards at data rates of 106, 212, or 424 kbit/s over distances of up to 10 cm.

The PN532 is a highly integrated NFC controller IC manufactured by NXP Semiconductors, supporting ISO 14443A/B, MIFARE, and FeliCa protocols. It communicates with a host microcontroller over I2C, SPI, or UART. In this project, the PN532 is interfaced to the ESP32 WROOM32 via I2C at 400 kHz (Fast Mode). The PN532's IRQ output pin signals the host that a card has been detected — a mechanism exploited by the interrupt-driven NFC implementation described in Chapter 4.

Each ISO 14443A card has a factory-programmed Unique Identifier (UID) of 4, 7, or 10 bytes. The UID is immutable and serves as the card's unique identifier for the purpose of vehicle check-in and check-out in this system. MIFARE Classic 1K cards (the most common variant) use 4-byte UIDs, which are represented in this project as colon-delimited hexadecimal strings (e.g., 'A1:B2:C3:D4').

## 2.5 Machine Learning-Based License Plate Recognition

Automatic License Plate Recognition (ALPR), also known as License Plate Recognition (LPR), is a computer vision technology that automatically extracts the alphanumeric characters of a vehicle's license plate from a photographic or video image. Modern LPR systems employ convolutional neural network (CNN) architectures for both plate detection (localizing the plate region within the image) and character segmentation and recognition (reading the individual characters).

Running a full CNN-based LPR pipeline directly on an ESP32-CAM is not feasible: the ESP32 has no hardware neural network accelerator, and even heavily quantized models would exhaust available memory and produce unacceptably long inference times. This project therefore delegates LPR to Plate Recognizer (platerecognizer.com), a commercial cloud API that accepts JPEG image uploads and returns detected plate strings with confidence scores and vehicle region information. To offload the TLS handshake overhead from the memory-constrained ESP32-CAM, the API call is proxied through the FastAPI backend server.

The LPR pipeline in this project implements confidence thresholding and Vietnamese license plate format validation via regular expression matching, ensuring that only high-quality recognition results are used for parking decisions. This hybrid approach — embedded capture, cloud inference, embedded decision — represents a pragmatic and increasingly common pattern in production IoT-ML systems.

## 2.6 MQTT Protocol for Embedded Messaging

MQTT (Message Queuing Telemetry Transport) is a publish-subscribe messaging protocol designed specifically for constrained devices and low-bandwidth, high-latency networks. Standardized by OASIS as MQTT 3.1.1 and 5.0, it operates over TCP/IP and uses a lightweight binary packet format with per-message overhead as small as 2 bytes.

MQTT's publish-subscribe model decouples producers and consumers: a publisher sends a message to a named topic on a central broker, and all clients subscribed to that topic receive a copy. This decoupling is architecturally important for this project: the NFC node can publish a trigger message without knowing the IP address of the camera node; the camera node receives it independently of the NFC node's state. MQTT supports three Quality of Service levels: QoS 0 (at-most-once, fire and forget), QoS 1 (at-least-once, acknowledged), and QoS 2 (exactly-once, two-phase commit). This project uses QoS 0 for most messages, relying on application-level retry logic for reliability.

The PubSubClient Arduino library provides the MQTT client implementation used on both the ESP32-CAM and NFC Node firmware. The broker in the prototype is Mosquitto, running on the same laptop as the FastAPI server.

## 2.7 Related Work

Several commercial and academic smart parking systems have been proposed and deployed. Conventional commercial systems such as those by Amano, Skidata, and Scheidt & Bachmann employ dedicated RFID readers, proprietary barrier controllers, and centralized management software — systems that are highly reliable but prohibitively expensive for small-scale deployments. Academic prototype systems have explored Raspberry Pi-based LPR (Anagnostopoulos et al., 2006), Arduino-based RFID parking (Idris et al., 2009), and ESP8266-based IoT parking monitoring (Waze integration, 2017). However, most existing prototypes either sacrifice real-time performance by delegating all logic to a server, or sacrifice scalability by centralizing hardware into a single node. The multi-node, MQTT-mediated architecture of this project with on-device decision making and FreeRTOS scheduling represents a more sophisticated approach that better balances autonomy, scalability, and embedded system engineering depth.

# CHAPTER 3: SYSTEM DESIGN AND ARCHITECTURE

## 3.1 Overall System Architecture

The Smart Parking Management System adopts a distributed, multi-node architecture that deliberately separates physical and functional concerns across three classes of specialized nodes. This separation achieves several engineering objectives simultaneously: it prevents resource contention between NFC polling, PWM servo actuation, camera DMA, and network communication; it isolates faults such that the failure of one node does not cascade across the system; and it allows each node to be updated or replaced independently. Figure 3.1 illustrates the physical and logical topology of the system.

All nodes communicate over a shared IEEE 802.11n 2.4 GHz WiFi local area network. The MQTT broker (Mosquitto) and HTTP backend server co-locate on the Controller Node (a laptop or Raspberry Pi). Nodes are assigned static IP addresses via DHCP reservation to ensure predictable routing. The total round-trip communication latency on the local area network is measured to be under 10 ms, making it negligible relative to the application-level timing requirements.

Figure 3.1 — Physical and Logical Network Topology of the Smart Parking System

| Node | Hardware | Primary Function | Network Role |
| --- | --- | --- | --- |
| NFC & Barrier | ESP32 WROOM32 + PN532 + 2× SG90 | Card reading, gate actuation, parking table | MQTT Publisher + Subscriber |
| Entry Camera | ESP32-CAM (OV2640 + PSRAM) | Capture entry image, run LPR, stream video | MQTT Subscriber + HTTP Client |
| Exit Camera | ESP32-CAM (OV2640 + PSRAM) | Capture exit image, run LPR, stream video | MQTT Subscriber + HTTP Client |
| Controller | Laptop (FastAPI + Mosquitto) | Session management, dashboard, admin override | MQTT Broker + HTTP Server |

## 3.2 Hardware Design

### 3.2.1 NFC and Barrier Control Node

The NFC and Barrier Control Node is built around the ESP32 WROOM32 module, chosen for its dual-core processing capability, integrated WiFi, hardware LEDC PWM peripheral, and ample GPIO count to simultaneously interface with the PN532 over I2C and drive two servo motors. The PN532 NFC controller module is wired in I2C mode (configured via solder jumpers on the module) with SDA on GPIO21 and SCL on GPIO22, using the ESP32's hardware I2C controller at 400 kHz. The two SG90 servo motors (entry barrier and exit barrier) are connected to GPIO26 and GPIO27 respectively, driven by the ESP32 LEDC peripheral.

The PN532 additionally exposes an IRQ (interrupt request) output pin, which is pulled low when a card is detected. This pin is connected to a general-purpose input of the ESP32 configured for falling-edge interrupt detection, enabling the interrupt-driven NFC design described in Section 4.2. An on-board status LED on GPIO2 provides visual feedback for card detection events and access denial conditions.

| Peripheral | GPIO | Interface | Configuration |
| --- | --- | --- | --- |
| PN532 SDA | GPIO21 | I2C | 400 kHz Fast Mode, pull-up 4.7kΩ |
| PN532 SCL | GPIO22 | I2C | 400 kHz Fast Mode, pull-up 4.7kΩ |
| PN532 IRQ | GPIO34 | Digital Input | INPUT_PULLUP, FALLING edge interrupt |
| Servo Entry | GPIO26 | LEDC PWM | 50 Hz, 16-bit, Channel 0 |
| Servo Exit | GPIO27 | LEDC PWM | 50 Hz, 16-bit, Channel 1 |
| Status LED | GPIO2 | Digital Output | Active HIGH, onboard LED |

### 3.2.2 Camera Nodes (Entry and Exit)

Both camera nodes use the AI-Thinker ESP32-CAM module, which integrates the ESP32-S chip, an OV2640 CMOS image sensor, 4 MB of SPI PSRAM, and a microSD card slot on a compact PCB. The OV2640 connects to the ESP32 via a fixed parallel interface (8-bit DVP), with clock, sync, and control signals on dedicated GPIO pins that are not user-configurable. The on-board flash LED on GPIO4 is repurposed as a capture illumination flash and an error indicator (blinking at 5 Hz when the camera subsystem enters the error state).

The two camera nodes run identical firmware, differentiated only by a compile-time flag NODE_ROLE (0 = Entry, 1 = Exit) which selects the MQTT topic to subscribe to and determines whether the horizontal mirror correction is applied.

### 3.2.3 Component Selection Rationale

| Component | Selected | Key Reason | Alternative Considered |
| --- | --- | --- | --- |
| MCU (NFC Node) | ESP32 WROOM32 | Dual-core, WiFi, LEDC PWM, I2C hardware, FreeRTOS | Arduino Uno: no WiFi, 2KB RAM; STM32: higher cost and complexity |
| MCU (Camera) | ESP32-CAM | OV2640 + PSRAM on-board, LEDC clock for camera, ecosystem maturity | Raspberry Pi Zero W: higher cost, Linux boot latency |
| NFC Reader | PN532 (I2C) | ISO 14443A support, IRQ pin for interrupt-driven design, stable Adafruit library | MFRC522: SPI only, no IRQ output for interrupt use |
| Camera Sensor | OV2640 (on-board) | Integrated with ESP32-CAM, JPEG hardware encoder, configurable resolution | OV5642: higher resolution but 5MP overkill for plate capture |
| Barrier Motor | SG90 Servo | 50Hz PWM standard, light weight, adequate torque for model barrier | Stepper motor: requires dedicated driver IC, more complex control |
| Comm. Protocol | MQTT (PubSubClient) | Pub-sub decoupling, lightweight binary format, broker handles routing | Direct HTTP: requires knowing target IP, polling-based, higher overhead |

## 3.3 Network and Communication Architecture

### 3.3.1 MQTT Topic Hierarchy

The system defines a structured MQTT topic hierarchy under the root namespace xdhtn/parking/. This namespace convention prevents topic collision with other MQTT services that may share the broker and allows straightforward access control rule definition in production deployments. The following topics are used:

| Topic | Publisher | Subscriber(s) | Payload | Purpose |
| --- | --- | --- | --- | --- |
| xdhtn/parking/trigger/entry | NFC Node | Entry Camera | UID string | Trigger entry capture |
| xdhtn/parking/trigger/exit | NFC Node | Exit Camera | UID string | Trigger exit capture |
| xdhtn/parking/result/entry | Entry Camera | NFC Node | JSON {card_id, plate, success} | LPR result for entry |
| xdhtn/parking/result/exit | Exit Camera | NFC Node | JSON {card_id, plate, success} | LPR result for exit |
| xdhtn/parking/override/entry | Server (admin) | NFC Node | 'override' | Manual gate open |
| xdhtn/parking/override/exit | Server (admin) | NFC Node | 'override' | Manual gate open |

### 3.3.2 HTTP/REST for Log Uploads

Following successful LPR processing, each camera node uploads a structured log record to the backend server via HTTP POST. Entry logs are sent as multipart/form-data, including the card UID, detected plate string, LPR success flag, and the JPEG image. Exit logs are sent as either multipart (with exit image) or JSON-only (if image capture was not triggered for exit). This design ensures the server has a complete audit record of every vehicle event, including photographic evidence at entry.

### 3.3.3 Server-Sent Events for Dashboard Push

The web dashboard receives real-time updates via Server-Sent Events (SSE), a W3C standard for server-to-client push over a persistent HTTP connection. Unlike WebSocket, SSE is unidirectional (server to client only), which is sufficient for this application — the dashboard only receives events, never sends data back over the SSE channel. SSE integrates naturally with FastAPI's StreamingResponse and async generators, and is natively supported by all modern browsers without additional libraries. Every parking event (entry, exit, barrier override, error) is broadcast to all connected SSE clients within milliseconds of occurring.

### 3.3.4 MJPEG for Live Video Streaming

Each ESP32-CAM node serves a live MJPEG (Motion JPEG) video stream on port 81 at the /stream endpoint, implemented using the ESP-IDF esp_http_server component. MJPEG is a simple streaming format where each video frame is sent as a complete JPEG image within a multipart HTTP response, delimited by MIME boundary strings. While less bandwidth-efficient than modern video codecs (H.264, VP8), MJPEG requires no decoder complexity on the receiving end and is supported natively in HTML <img> and <video> tags. The backend server proxies these streams at /stream/entry and /stream/exit, allowing the dashboard to display both camera feeds through a single domain without cross-origin issues.

## 3.4 Software Architecture

### 3.4.1 Four-Layer Software Model

The firmware is organized into four conceptual layers, following the principle of separation of concerns. This layering ensures that changes to hardware configuration do not require modifications to business logic, and that the task layer can be tested independently of the physical hardware.

┌──────────────────────────────────────────────────────────────────┐
│  APPLICATION LAYER                                                │
│  Parking business rules: entry/exit FSM, session management,     │
│  LPR validation, barrier decision logic (NFC Node gParkingTable) │
├──────────────────────────────────────────────────────────────────┤
│  SERVICE / COMMUNICATION LAYER                                    │
│  MQTT pub/sub (PubSubClient), HTTP multipart upload (HTTPClient), │
│  FastAPI REST + SSE + MJPEG proxy (Python backend)               │
├──────────────────────────────────────────────────────────────────┤
│  FIRMWARE TASK LAYER  (FreeRTOS)                                  │
│  ESP32-CAM: taskMQTT | taskCamera | taskLPR | taskLog | taskStream│
│  NFC Node:  taskNFC  | taskServo  | [Software Timer: gateClose]  │
├──────────────────────────────────────────────────────────────────┤
│  HAL / DRIVER LAYER                                               │
│  esp_camera (I2S DMA), Adafruit_PN532 (I2C), LEDC PWM,           │
│  WiFi/TCP stack, PSRAM heap_caps API                              │
└──────────────────────────────────────────────────────────────────┘

Figure 3.2 — Four-Layer Software Architecture

### 3.4.2 Firmware Segregation via PlatformIO Build System

A single source tree supports three distinct firmware binaries through the PlatformIO build system's build_src_filter mechanism. Each firmware binary includes only the source files relevant to its role, preventing symbol conflicts and reducing binary size. Three PlatformIO environments are defined: entry_node (ESP32-CAM, NODE_ROLE=0), exit_node (ESP32-CAM, NODE_ROLE=1), and nfc_node (ESP32 WROOM32). The NODE_ROLE preprocessor define is propagated via compiler flags (-DNODE_ROLE=0 or -DNODE_ROLE=1), allowing a single camera firmware source to produce two role-specific binaries.

### 3.4.3 Finite State Machine Design

The ESP32-CAM firmware implements an explicit Finite State Machine (FSM) to govern the sequencing of camera capture operations. The FSM state is stored in a volatile global variable (gState, of type SystemState_t) protected by a mutex (xStateMutex). The FSM has six states:

| State | Meaning | Transitions To | Duration |
| --- | --- | --- | --- |
| STATE_IDLE | System ready, awaiting trigger | STATE_CARD_READ on MQTT trigger | Indefinite |
| STATE_CARD_READ | MQTT trigger received, camera arming | STATE_CAPTURING immediately | <100 ms |
| STATE_CAPTURING | Camera warm-up and frame capture in progress | STATE_LPR_PENDING on success; STATE_IDLE on failure | ~300 ms |
| STATE_LPR_PENDING | LPR API call in progress | STATE_GATE_OPEN on success; STATE_LPR_FAIL on failure | 500–3000 ms |
| STATE_GATE_OPEN | LPR succeeded, MQTT result published, gate commanded | STATE_IDLE after publish | <100 ms |
| STATE_LPR_FAIL | LPR failed or confidence below threshold | STATE_IDLE after failure handling | <100 ms |
| STATE_ERROR | Camera hardware initialization failure | No automatic recovery; LED blinks | Indefinite |

### 3.4.4 Data Flow: Entry and Exit Sequences

The complete data flow for a vehicle entry event, from card presentation to barrier opening, is shown in Figure 3.3 below. The exit flow is symmetric.

ENTRY SEQUENCE:
  [Vehicle Driver presents NFC card to PN532]
        │ PN532 IRQ pin falls LOW
        ▼
  taskNFC (Core 1)  — xSemaphoreGiveFromISR() in ISR
        │ reads UID, checks gParkingTable (card NOT in table → ENTRY)
        │ publishes to xdhtn/parking/trigger/entry
        ▼
  Entry ESP32-CAM  — taskMQTT receives trigger, xQueueOverwrite(xNfcQueue)
        │
        ▼
  taskCamera (Core 1) — warm-up + capture, PSRAM alloc, xQueueSend(xCaptureQueue)
        │
        ▼
  taskLPR (Core 0) — HTTP POST to /api/proxy-lpr, parse result, validate plate
        │ publishes to xdhtn/parking/result/entry {plate, success=1}
        │
        ▼
  NFC Node — mqttCallback receives result
        │ tableInsert(uid, plate) → gParkingTable updated
        │ xQueueSend(xGateQueue, GATE_ENTRY)
        ▼
  taskServo — servoSet(ENTRY_PIN, 90°) → vTaskDelay(5000ms) → servoSet(0°)
        │
        ▼
  [Barrier opens for 5 seconds, then closes automatically]

Figure 3.3 — Complete Entry Event Data Flow Sequence

The exit sequence is the mirror image: the NFC node detects that the scanned card UID is present in gParkingTable (vehicle currently parked), triggers the exit camera via MQTT, the exit camera captures and runs LPR, the NFC node validates that the detected plate matches the stored entry plate, calls tableMatchAndRemove() to remove the record, and commands the exit barrier to open. If the plate does not match, or if LPR fails, the gate remains closed and the access denial LED pattern is triggered.

# CHAPTER 4: SYSTEM IMPLEMENTATION

## 4.1 Development Environment and Build System

The firmware for both node types is developed using the PlatformIO IDE extension for Visual Studio Code, targeting the Arduino framework on top of Espressif's ESP-IDF. PlatformIO provides dependency management, build flag propagation, multi-environment support, serial monitor, and firmware upload — all without requiring manual toolchain installation.

Three build environments are defined in platformio.ini: entry_node and exit_node (both targeting the esp32cam board with platform-espressif32), and nfc_node (targeting the esp32dev board). The camera environments inherit common settings from a base environment env_cam_base, using the extends keyword. Build flags include -DBOARD_HAS_PSRAM and -mfix-esp32-psram-cache-issue to enable PSRAM support and apply the known ESP32 PSRAM cache errata fix.

; platformio.ini — abbreviated
[env_cam_base]
platform  = espressif32
board     = esp32cam
framework = arduino
board_build.partitions = huge_app.csv
lib_deps  = bblanchon/ArduinoJson@^7.0.0
            knolleary/PubSubClient@^2.8
build_src_filter =
    +<*> -<nfc_node.cpp> -<nfc_task.cpp> -<http_task.cpp>
build_flags = -DCORE_DEBUG_LEVEL=3 -DBOARD_HAS_PSRAM
              -mfix-esp32-psram-cache-issue

[env:entry_node]
extends    = env_cam_base
build_flags = ${env_cam_base.build_flags} -DNODE_ROLE=0

[env:exit_node]
extends    = env_cam_base
build_flags = ${env_cam_base.build_flags} -DNODE_ROLE=1

[env:nfc_node]
platform  = espressif32
board     = esp32dev
framework = arduino
lib_deps  = adafruit/Adafruit PN532@^1.3.4
            knolleary/PubSubClient@^2.8
            bblanchon/ArduinoJson@^7.0.0
build_src_filter = +<nfc_node.cpp>

Figure 4.1 — PlatformIO Configuration File

The build_src_filter directive is particularly important: it ensures that the ESP32-CAM firmware binary does not include any NFC or servo code, and the NFC Node binary does not include any camera or LPR code. This eliminates symbol conflicts and produces smaller, role-specific binaries. The shared header files (config.h and system_state.h) are included by all targets and provide the common type definitions and configuration constants.

## 4.2 Interrupt-Driven NFC Reader Implementation

The NFC reader implementation is one of the most technically significant components of this project from an embedded systems perspective. Two approaches were designed and analyzed: polling-based NFC reading, and interrupt-driven (IRQ-triggered) NFC reading. The polling approach was used in early prototype versions for simplicity and stability; the interrupt-driven approach represents the architecturally preferred design and is documented here as the primary implementation.

### 4.2.1 Interrupt Service Routine Design

The PN532 NFC controller asserts its IRQ output pin LOW when a card has been detected and data is ready to be read. By connecting this pin to a GPIO input on the ESP32 and attaching a FALLING-edge hardware interrupt, the taskNFC FreeRTOS task can sleep indefinitely (consuming zero CPU) and be woken instantly the moment a card is presented — eliminating the CPU overhead and latency of polling.

The ISR function must be placed in IRAM (Internal RAM) rather than flash memory, because on the ESP32, flash is accessed via SPI with cache and cannot be reliably read during certain WiFi operations (cache invalidation). The IRAM_ATTR attribute instructs the linker to place the function in the 192 KB IRAM region that is always directly accessible. The ISR uses xSemaphoreGiveFromISR(), the ISR-safe variant of the semaphore API that can safely be called from interrupt context:

// nfc_task.cpp — Interrupt Service Routine
static SemaphoreHandle_t xIrqSemaphore = nullptr;

static void IRAM_ATTR pn532IrqHandler() {
    // Must be ISR-safe: no floating point, no heap, no blocking
    BaseType_t xHigherPriorityTaskWoken = pdFALSE;
    xSemaphoreGiveFromISR(xIrqSemaphore, &xHigherPriorityTaskWoken);
    // If giving the semaphore unblocked a higher-priority task,
    // request an immediate context switch at ISR exit.
    portYIELD_FROM_ISR(xHigherPriorityTaskWoken);
}

Figure 4.2 — ISR Function with IRAM_ATTR and ISR-Safe FreeRTOS API

The portYIELD_FROM_ISR() call at the end of the ISR is critical for correct real-time behavior. Without it, the task woken by the semaphore give would not be scheduled until the next FreeRTOS tick (up to 1 ms later). With portYIELD_FROM_ISR(), if xHigherPriorityTaskWoken is set to pdTRUE by xSemaphoreGiveFromISR() (indicating that the woken task has a higher priority than the currently preempted task), the CPU immediately switches to the higher-priority task upon ISR exit — achieving true interrupt-to-task response times in the microsecond range.

### 4.2.2 Race Condition Analysis and Prevention

A subtle but critical race condition exists in interrupt-driven NFC designs that is not present in polling implementations. If the ISR fires between the point where xIrqSemaphore is declared and where xSemaphoreCreateBinary() initializes it, the ISR will attempt to call xSemaphoreGiveFromISR() on a null handle, resulting in a null pointer exception and system crash.

The correct fix is to ensure that the semaphore is fully created before the interrupt is attached:

// taskNFC() — Correct initialization ordering
void taskNFC(void *pvParameters) {
    // Step 1: Initialize I2C and PN532
    Wire.begin(PN532_SDA_PIN, PN532_SCL_PIN, 400000UL);
    pn532.begin();
    // ... (PN532 firmware version check and SAMConfig) ...

    // Step 2: Create semaphore BEFORE attaching interrupt
    xIrqSemaphore = xSemaphoreCreateBinary();
    configASSERT(xIrqSemaphore); // crash on allocation failure

    // Step 3: Configure GPIO and arm PN532 for detection
    pinMode(PN532_IRQ_PIN, INPUT_PULLUP);
    pn532.startPassiveTargetIDDetection(PN532_MIFARE_ISO14443A);

    // Step 4: Attach interrupt LAST — after semaphore exists
    attachInterrupt(
        digitalPinToInterrupt(PN532_IRQ_PIN),
        pn532IrqHandler,
        FALLING
    );
    // Now IRQ can fire safely
    ...
}

Figure 4.3 — Race-Condition-Free Interrupt Initialization Sequence

### 4.2.3 Main Task Loop and Debounce

With the interrupt infrastructure in place, the main NFC task loop is elegantly simple. It blocks indefinitely on the semaphore, wakes only when a card is detected, reads the UID, processes the event, and then implements a debounce loop to wait until the card is physically removed before re-arming for the next detection:

while (true) {
    // Re-arm PN532 for the next card
    pn532.startPassiveTargetIDDetection(PN532_MIFARE_ISO14443A);

    // Block until IRQ fires (card present) or 5-second timeout
    if (xSemaphoreTake(xIrqSemaphore,
                       pdMS_TO_TICKS(NFC_READ_TIMEOUT_MS)) != pdTRUE) {
        continue; // timeout — no card detected, re-arm silently
    }

    // Read UID from PN532
    bool success = pn532.readDetectedPassiveTargetID(
                       evt.uid, &evt.uidLen);
    if (!success || evt.uidLen == 0) {
        Serial.println("[NFC] Read failed — spurious IRQ");
        continue;
    }

    // ... process event (publish MQTT, update parking table) ...

    // Debounce: wait for card removal (up to 3 seconds)
    uint32_t debounceStart = millis();
    while (millis() - debounceStart < 3000) {
        pn532.startPassiveTargetIDDetection(PN532_MIFARE_ISO14443A);
        if (xSemaphoreTake(xIrqSemaphore, pdMS_TO_TICKS(200)) != pdTRUE)
            break; // no IRQ in 200ms → card removed
        pn532.readDetectedPassiveTargetID(tmpUid, &tmpLen);
    }
}

Figure 4.4 — Interrupt-Driven NFC Task Main Loop with Debounce

## 4.3 Hardware Timer and PWM Implementation

### 4.3.1 LEDC PWM for Servo Motor Control

The SG90 servo motor requires a 50 Hz PWM signal (20 ms period) with a pulse width between approximately 0.5 ms and 2.4 ms to control its angular position. A 0.5 ms pulse corresponds to 0° (gate closed), and a 1.5 ms pulse corresponds to 90° (gate fully open). The ESP32 LEDC peripheral generates this signal entirely in hardware, with a configurable frequency and duty cycle resolution. Using 16-bit resolution provides 65,536 discrete duty cycle steps, giving sub-millisecond PWM precision.

The angular position to duty cycle conversion is implemented by the angleToDuty() function, calibrated for the 16-bit / 50 Hz configuration:

// nfc_node.cpp — Servo PWM configuration and control
#define SERVO_FREQ      50    // 50 Hz — standard servo RC frequency
#define SERVO_RES       16    // 16-bit resolution → 65536 steps
#define SERVO_OPEN_DEG  90    // gate open angle
#define SERVO_CLOSE_DEG  0    // gate closed angle

// Period = 1/50Hz = 20ms = 20,000 µs
// At 16-bit: 1 step = 20,000µs / 65536 ≈ 0.305 µs/step
// 0.5ms pulse  = 0.5ms  / 0.305µs ≈ 1638 steps  (0°)
// 1.5ms pulse  = 1.5ms  / 0.305µs ≈ 4915 steps  (90°)
// 2.4ms pulse  = 2.4ms  / 0.305µs ≈ 7864 steps  (180°)

static uint32_t angleToDuty(int angle) {
    // Linear interpolation between 0° (duty=1638) and 180° (duty=8192)
    return (uint32_t)(1638 + (angle * (8192 - 1638)) / 180);
}

static void servoSet(uint8_t pin, int angle) {
    ledcWrite(pin, angleToDuty(angle));
}

// LEDC initialization (Arduino Core v3 unified API)
ledcAttach(SERVO_ENTRY_PIN, SERVO_FREQ, SERVO_RES);
ledcAttach(SERVO_EXIT_PIN,  SERVO_FREQ, SERVO_RES);
servoSet(SERVO_ENTRY_PIN, SERVO_CLOSE_DEG); // ensure gate starts closed

Figure 4.5 — LEDC PWM Servo Control Implementation

Once ledcAttach() and the initial ledcWrite() are called, the LEDC hardware timer autonomously maintains the PWM signal on the GPIO pin. No CPU cycles are consumed to sustain the signal — the hardware timer peripheral runs independently. This is a fundamental advantage of hardware timer-based PWM over software bit-banging approaches, which would require the CPU to toggle the GPIO pin every half-period and would be incompatible with a multitasking RTOS environment.

### 4.3.2 FreeRTOS Software Timer for Gate Auto-Close

In early firmware versions, gate auto-close was implemented using a one-shot FreeRTOS task that called servoSet(OPEN) and then blocked for 5 seconds with vTaskDelay(SERVO_OPEN_MS) before calling servoSet(CLOSE) and deleting itself. While functional, this approach wastes a task stack (2 KB minimum) for the entire 5-second period and unnecessarily inflates the task count.

The improved implementation uses a FreeRTOS Software Timer — a timer that invokes a callback function in the context of the Timer Service Task after a specified delay, consuming no dedicated task stack. The gate open command sets the servo to 90° and starts a 5-second one-shot timer; when the timer fires, the callback sets the servo back to 0°:

// nfc_node.cpp — FreeRTOS Software Timer for gate auto-close
static TimerHandle_t gEntryCloseTimer = nullptr;
static TimerHandle_t gExitCloseTimer  = nullptr;

// Timer callback — runs in Timer Service Task context
static void entryCloseCallback(TimerHandle_t xTimer) {
    servoSet(SERVO_ENTRY_PIN, SERVO_CLOSE_DEG);
    digitalWrite(LED_PIN, LOW);
    Serial.println("[SERVO] Entry gate closed by timer.");
}

static void exitCloseCallback(TimerHandle_t xTimer) {
    servoSet(SERVO_EXIT_PIN, SERVO_CLOSE_DEG);
    digitalWrite(LED_PIN, LOW);
    Serial.println("[SERVO] Exit gate closed by timer.");
}

// In setup():
gEntryCloseTimer = xTimerCreate(
    "entryClose",            // Timer name (debug only)
    pdMS_TO_TICKS(5000),      // 5-second period
    pdFALSE,                  // pdFALSE = one-shot (not periodic)
    nullptr,                  // Timer ID (not used)
    entryCloseCallback        // Callback function
);

// When gate opens:
static void openGate(uint8_t pin, TimerHandle_t closeTimer) {
    servoSet(pin, SERVO_OPEN_DEG);
    digitalWrite(LED_PIN, HIGH);
    xTimerStart(closeTimer, 0); // start 5s countdown, non-blocking
    // Returns immediately — gate closes automatically in 5 seconds
}

Figure 4.6 — FreeRTOS Software Timer for Non-Blocking Gate Auto-Close

This implementation eliminates the need for one-shot tasks entirely, reduces memory usage by 2× stack allocation, and makes the gate timing more predictable — the FreeRTOS timer service uses the same SysTick tick count as all other RTOS operations, so the 5-second timeout is as accurate as the FreeRTOS tick rate (±1 tick = ±1 ms).

### 4.3.3 Hardware Watchdog Timer for Fault Recovery

Embedded systems that run unattended in real environments must handle hardware faults gracefully. One class of fault that is particularly dangerous is the I2C bus lockup: if a transaction is interrupted (e.g., by a power glitch or electromagnetic interference), the PN532 or the ESP32 I2C controller may leave the bus in an indeterminate state, causing all subsequent I2C operations to block indefinitely and starving the NFC task of progress.

The ESP32 hardware watchdog timer provides a backstop against such lockups. A hardware timer is configured to fire after 8 seconds; the NFC task 'pets' (resets) the watchdog at the start of each iteration. If the task becomes stuck (e.g., in a blocking I2C call with no timeout), the watchdog fires and executes a recovery ISR that asserts a software reset of the I2C peripheral and the PN532 module — without resetting the entire ESP32 or losing the WiFi connection and parking table state:

// nfc_node.cpp — Hardware Watchdog Timer
static hw_timer_t *wdTimer = nullptr;
static volatile bool wdFired = false;

static void IRAM_ATTR watchdogISR() {
    wdFired = true;
    // Signal recovery: in the task loop, wdFired triggers I2C reinit
}

// In setup():
wdTimer = timerBegin(1, 80, true);    // Timer 1, prescaler 80 → 1MHz tick
timerAttachInterrupt(wdTimer, &watchdogISR, true); // edge-triggered
timerAlarmWrite(wdTimer, 8000000, false); // 8,000,000 µs = 8 seconds
timerAlarmEnable(wdTimer);

// In taskNFC main loop:
while (true) {
    timerWrite(wdTimer, 0);  // pet the watchdog — reset countdown

    if (wdFired) {
        wdFired = false;
        Serial.println("[WDT] Watchdog fired — reinitializing I2C/PN532");
        Wire.end();
        vTaskDelay(pdMS_TO_TICKS(100));
        Wire.begin(NFC_SDA_PIN, NFC_SCL_PIN, 400000UL);
        pn532.begin();
        pn532.SAMConfig();
        timerAlarmEnable(wdTimer); // re-arm watchdog
    }
    // ... rest of NFC task loop ...
}

Figure 4.7 — Hardware Watchdog Timer for I2C Bus Fault Recovery

## 4.4 FreeRTOS Task Architecture

### 4.4.1 Complete Task Map

The complete set of FreeRTOS tasks across both firmware images, with their stack sizes, priorities, core assignments, and roles, is documented in Tables 4.1 and 4.2.

| Task | Function | Stack (B) | Priority | Core | Role |
| --- | --- | --- | --- | --- | --- |
| taskMQTT | taskMQTT() | 4096 | 2 | 0 | MQTT client loop, receive trigger, dispatch NfcEvent to camera task |
| taskCamera | taskCamera() | 8192 | 3 | 1 | Camera init, warm-up, frame capture, PSRAM buffer management |
| taskLPR | taskLPR() | 8192 | 2 | 0 | LPR API call, JSON parse, plate validation, MQTT result publish |
| taskLogUpload | taskLogUpload() | 6144 | 1 | 0 | HTTP log upload to backend server, image and JSON payloads |
| taskStream | taskStreamServer() | 6144 | 2 | 1 | MJPEG httpd server startup, then self-deletes; httpd runs on IDF task |

Table 4.1 — ESP32-CAM FreeRTOS Task Map

| Task | Function | Stack (B) | Priority | Core | Role |
| --- | --- | --- | --- | --- | --- |
| taskNFC | taskNFC() | 4096 | 3 | 1 | IRQ-driven NFC read, parking table lookup, MQTT trigger publish |
| taskServo | taskServo() | 3072 | 2 | 0 | Receive gate commands from xGateQueue, actuate servo, trigger close timer |

Table 4.2 — NFC Node FreeRTOS Task Map

### 4.4.2 Core Pinning Rationale

The decision of which core to pin each task to is not arbitrary — it has direct consequences for system latency and correctness. The ESP32's WiFi radio stack runs on Core 0 and can consume up to 30–40% of Core 0's CPU time during active transmission or reception. Camera DMA operations are sensitive to CPU stalls: if the CPU is unable to process the DMA interrupt in time, frames may be dropped or corrupted.

By pinning taskCamera and the MJPEG stream task to Core 1, these latency-sensitive camera operations are completely isolated from WiFi interrupt interference. All network tasks (taskMQTT, taskLPR, taskLogUpload) are pinned to Core 0, where they naturally share CPU time with the TCP/IP stack and benefit from cache locality — reducing the overhead of cross-core context switches that would otherwise occur if a network task pinned to Core 1 needed to access TCP/IP stack data.

### 4.4.3 Priority Assignment and Priority Inversion Analysis

Task priorities are assigned based on timing criticality and dependency ordering. taskCamera has the highest priority (3) because it sits on the critical path between trigger reception and LPR submission — any delay in capture increases total system latency. taskMQTT and taskLPR share priority 2, ensuring that MQTT messages are processed promptly and LPR runs as soon as a frame is available. taskLogUpload has priority 1 (lowest non-idle) because log uploads are best-effort: a delayed log upload is acceptable, whereas a delayed gate opening is not.

Priority inversion — a situation where a high-priority task is indirectly blocked by a low-priority task via a shared mutex — is a risk in any multi-task system using mutexes. In this firmware, xStateMutex and xCameraMutex are implemented as proper FreeRTOS mutexes (xSemaphoreCreateMutex, not xSemaphoreCreateBinary), which automatically provide priority inheritance. If taskCamera (priority 3) attempts to acquire xCameraMutex while it is held by the MJPEG stream handler (which runs in the lower-priority IDF httpd task), the IDF httpd task's effective priority is temporarily elevated to 3, allowing it to complete quickly and release the mutex without further delay. This prevents the priority inversion from causing a live-lock condition.

### 4.4.4 Stack Size Determination and High-Water Mark Monitoring

Stack overflow is one of the most common causes of silent corruption and hard faults in FreeRTOS applications. Stack sizes must be large enough to accommodate the deepest call chain that any task might execute, including all intermediate function call frames, local variables, and processor register save areas. The following analysis guided the stack size assignments:

taskCamera (8192 B): Must accommodate the camera initialization call chain (esp_camera_init calls several IDF sub-functions), the PSRAM buffer allocation, and multiple nested conditional branches. 8192 B provides a comfortable margin.

taskLPR (8192 B): The JSON parsing path (ArduinoJson JsonDocument) allocates significant stack for its internal state machine. HTTP multipart body construction also requires intermediate buffers. 8192 B is the practical minimum.

taskMQTT (4096 B): The PubSubClient library's callback mechanism is relatively shallow. 4096 B is adequate.

taskLogUpload (6144 B): Multipart HTTP body construction with image data requires intermediate header and footer buffers. 6144 B provides margin.

taskNFC (4096 B): I2C read operations and JSON parsing are the deepest call chains. 4096 B is adequate.

To verify that stack allocations are sufficient at runtime, the system implements a periodic stack high-water mark reporting routine that calls uxTaskGetStackHighWaterMark() for each task handle and logs the remaining free stack words to the serial monitor every 30 seconds:

// main.cpp — Stack high-water mark monitoring in loop()
static uint32_t lastStackReport = 0;

if (millis() - lastStackReport > 30000) {
    lastStackReport = millis();
    Serial.printf("[STACK] taskCamera:    %u words free\n",
        uxTaskGetStackHighWaterMark(hCameraTask));
    Serial.printf("[STACK] taskLPR:       %u words free\n",
        uxTaskGetStackHighWaterMark(hLprTask));
    Serial.printf("[STACK] taskMQTT:      %u words free\n",
        uxTaskGetStackHighWaterMark(hMqttTask));
    Serial.printf("[STACK] taskLogUpload: %u words free\n",
        uxTaskGetStackHighWaterMark(hLogTask));
}

Figure 4.8 — Runtime Stack High-Water Mark Monitoring

A high-water mark of zero indicates an imminent stack overflow. The monitoring system alerts engineers before failures occur, allowing stack sizes to be increased proactively before deployment.

## 4.5 Inter-Task Communication and Synchronization

### 4.5.1 Queue Pipeline Architecture

Data flows through the ESP32-CAM firmware as a pipeline of FreeRTOS queues, each with a depth of 1 except the log queue (depth 4). The depth-1 design reflects the system's single-vehicle-at-a-time processing model: if a new MQTT trigger arrives while a capture is in progress, the existing state machine rejects it, preventing queue buildup and ensuring ordered processing.

Queue pipeline data flow:

  taskMQTT   →  xNfcQueue (depth 1, NfcEvent_t)
                    │  xQueueOverwrite() — always replaces stale trigger
                    ▼
  taskCamera →  xCaptureQueue (depth 1, CaptureEvent_t)
                    │  xQueueSend() — blocks 500ms if full
                    ▼
  taskLPR    →  xLogQueue (depth 4, LprEvent_t)
                    │  xQueueSend() — non-blocking, drops if full
                    ▼
  taskLogUpload → POST /api/log/entry or /api/log/exit

Figure 4.9 — FreeRTOS Queue Pipeline Architecture

The use of xQueueOverwrite() for the NFC trigger queue (xNfcQueue) is a deliberate design choice. Because xNfcQueue has depth 1, xQueueSend() would block if the queue already contains an unprocessed event — an undesirable situation if the camera task is busy. xQueueOverwrite() instead atomically replaces the existing item with the new one, guaranteeing non-blocking behavior and ensuring that the most recent trigger is always the one processed. This is appropriate for trigger events where only the latest is meaningful.

### 4.5.2 Mutex-Protected Shared Resources

Two global resources are shared across multiple tasks and require mutex protection. The xStateMutex protects the gState FSM state variable, which is read and written by taskMQTT (state gate check), taskCamera (state update to CAPTURING), taskLPR (state update to LPR_PENDING), and the loop() function (state monitoring). The xCameraMutex serializes access to the esp_camera framebuffer API, which is used concurrently by taskCamera (capture) and the MJPEG httpd handler (streaming).

// Pattern: mutex-protected FSM state update
if (xSemaphoreTake(xStateMutex, pdMS_TO_TICKS(100)) == pdTRUE) {
    gState = STATE_CAPTURING;
    xSemaphoreGive(xStateMutex);
} else {
    // Timeout acquiring mutex — log and continue
    Serial.println("[CAM] State mutex timeout — state not updated");
}

// Pattern: mutex-protected camera framebuffer access
camera_fb_t *fb = nullptr;
if (xSemaphoreTake(xCameraMutex, pdMS_TO_TICKS(1000)) == pdTRUE) {
    fb = esp_camera_fb_get();   // acquires DMA buffer
    xSemaphoreGive(xCameraMutex);
}
// Always return DMA buffer — even on mutex timeout
if (fb) {
    // ... process frame ...
    if (xSemaphoreTake(xCameraMutex, pdMS_TO_TICKS(500)) == pdTRUE) {
        esp_camera_fb_return(fb);
        xSemaphoreGive(xCameraMutex);
    } else {
        esp_camera_fb_return(fb); // must return even without mutex
    }
}

Figure 4.10 — Mutex-Protected Shared Resource Access Patterns

### 4.5.3 Task Notifications for Camera-Ready Signaling

The MJPEG stream task (taskStream) must wait for taskCamera to complete camera initialization before it can begin serving frames. The original implementation used a volatile bool flag (gCameraReady) polled in a busy-wait loop. The improved implementation uses a FreeRTOS Task Notification: taskCamera sends a notification to taskStream immediately after successful initialization, and taskStream blocks on ulTaskNotifyTake() until the notification arrives. This eliminates the busy-wait loop and reduces CPU usage during the initialization phase:

// camera_task.cpp — notify stream task when camera is ready
TaskHandle_t hStreamTask = nullptr; // handle set in setup()

void taskCamera(void *pvParameters) {
    // ... camera initialization ...
    if (!camOk) { /* error handling */ return; }

    // Notify stream task — camera is ready to serve frames
    xTaskNotifyGive(hStreamTask);
    // ... camera main loop ...
}

// stream_server.cpp — block until notified
void taskStreamServer(void *pvParameters) {
    // Block indefinitely until taskCamera sends notification
    ulTaskNotifyTake(pdTRUE, portMAX_DELAY);
    // Camera is now initialized — safe to start MJPEG server
    startMjpegServer();
    // ...
}

Figure 4.11 — Task Notification for Camera-Ready Signaling

## 4.6 Camera Subsystem Implementation

### 4.6.1 OV2640 Initialization and Sensor Tuning

The OV2640 camera sensor is initialized using the esp_camera library, which abstracts the I2S DMA configuration, SCCB (I2C-compatible) sensor register programming, and framebuffer management. The camera is configured for SVGA resolution (800×600 pixels), JPEG pixel format (hardware-compressed by the OV2640's internal DSP), a quality factor of 12 (higher quality, larger files), and three framebuffers in PSRAM. After initialization, sensor registers are tuned for the parking gate environment: brightness +1 (compensate for variable outdoor lighting), contrast +1 (improve plate edge definition), auto white balance enabled, and horizontal mirror correction applied for the entry camera (mounted facing the driver, mirror effect compensated).

### 4.6.2 DMA-Based Frame Capture Pipeline

The heart of the camera subsystem is the DMA-based capture pipeline. When esp_camera_fb_get() is called, the following sequence occurs internally within the esp_camera/IDF layer, entirely without application CPU involvement:

The I2S peripheral receives pixel clock (PCLK) and data signals from the OV2640 via the DVP interface.

On each VSYNC pulse (frame start), a DMA channel is armed to transfer pixel data from the I2S FIFO into a pre-allocated PSRAM framebuffer.

The OV2640 DSP compresses the raw Bayer sensor data into JPEG on-chip. JPEG-compressed data is output on the DVP bus.

When HREF goes low at the end of the last line (frame complete), the DMA transfer is complete and an internal semaphore is given.

esp_camera_fb_get() returns the pointer to the completed JPEG data in PSRAM.

The CAMERA_GRAB_LATEST grab mode instructs the camera driver to always return the most recently completed frame, discarding any intermediate frames that accumulated while the application was processing the previous frame. This is essential for the MJPEG stream, which must always show the most current view, not a frame that is several hundred milliseconds old.

### 4.6.3 PSRAM Memory Management and Buffer Lifecycle

Correct PSRAM buffer lifecycle management is essential to prevent memory leaks that would cause the system to crash after a number of capture cycles. The buffer lifecycle is as follows:

PSRAM Buffer Lifecycle:

  [taskCamera]
  1. esp_camera_fb_get()          → DMA buffer in PSRAM (owned by camera driver)
  2. heap_caps_malloc(SPIRAM)      → application PSRAM buffer allocated
  3. memcpy(appBuf, dma.buf, len) → copy frame data to application buffer
  4. esp_camera_fb_return(dma)    → DMA buffer returned to camera driver pool
     (camera driver can now reuse this DMA slot for next frame)
  5. xQueueSend(xCaptureQueue, {appBuf, len, ...})
     → ownership of appBuf transferred to taskLPR

  [taskLPR]
  6. buildPlateApiBody(appBuf)    → HTTP multipart body in PSRAM allocated
  7. http.POST(body, bodyLen)     → image data sent to LPR API
  8. heap_caps_free(body)         → multipart body freed
  9. xQueueSend(xLogQueue, {appBuf, ...}) → appBuf ownership to taskLogUpload

  [taskLogUpload]
 10. buildEntryLog(appBuf)        → log multipart body in PSRAM allocated
 11. http.POST(logBody, logLen)   → log + image sent to backend server
 12. heap_caps_free(logBody)      → log body freed
 13. heap_caps_free(appBuf)       → APPLICATION PSRAM BUFFER FREED
     (memory returned to heap — no leak)

Figure 4.12 — Complete PSRAM Buffer Ownership and Lifecycle

### 4.6.4 Warm-Up Frame and Auto-Exposure Settle

When the camera capture is triggered, the OV2640's automatic exposure (AE) and automatic white balance (AWB) algorithms need one or two frame periods to stabilize after the initial trigger. If the first frame is captured immediately, it may be over or underexposed relative to the actual scene conditions. The firmware therefore captures one 'warm-up' frame (immediately discarded) and waits 150 ms before capturing the actual frame used for LPR processing. During this 150 ms, the flash LED is illuminated at full brightness to provide consistent lighting for the plate. The warm-up frame approach adds 200–250 ms to the total capture latency, but substantially improves LPR accuracy by ensuring well-exposed images.

## 4.7 License Plate Recognition Pipeline

### 4.7.1 Server-Proxied LPR API Call

The ESP32-CAM communicates with the Plate Recognizer API (api.platerecognizer.com) indirectly through the backend FastAPI server's /api/proxy-lpr endpoint. This proxy architecture serves two purposes: first, it offloads the TLS handshake and certificate validation to the backend server, which has far more RAM and CPU available than the ESP32-CAM; second, it keeps the API token on the server rather than in the ESP32 firmware, improving security. The ESP32-CAM sends an unencrypted HTTP POST to the local server, which forwards the image to Plate Recognizer over HTTPS and returns the JSON response.

### 4.7.2 Confidence Thresholding and Vietnamese Plate Validation

The Plate Recognizer API returns detection results as a JSON array, where each element contains the detected plate string, a recognition confidence score (0.0–1.0), and a vehicle detection confidence score. The firmware applies a minimum confidence threshold (PLATE_CONFIDENCE = 0.75) and validates the detected plate string against a regular expression pattern for Vietnamese license plate formats before accepting the result:

// lpr_task.cpp — Plate validation
#define PLATE_CONFIDENCE  0.75f
// Vietnamese plate formats:
// Standard 4-digit: 51A-123.45, 51AB-12345
// Regex covers: 2-digit province + 1-2 letter series + 4-5 digit number
#define PLATE_REGEX  "^[0-9]{2}[A-Z]{1,2}[0-9]{4,5}$"

static bool plateValid(const char *plate) {
    regex_t re;
    if (regcomp(&re, PLATE_REGEX, REG_EXTENDED | REG_NOSUB) != 0)
        return false;
    bool ok = (regexec(&re, plate, 0, nullptr, 0) == 0);
    regfree(&re);
    return ok;
}

// In callPlateApi():
if (conf >= PLATE_CONFIDENCE && plateValid(normalized)) {
    strlcpy(outPlate, normalized, plateSize);
    found = true; // accept this result
    break;
}

Figure 4.13 — LPR Confidence Thresholding and Format Validation

### 4.7.3 LPR Result Publication via MQTT

After the LPR result is determined (success with plate string, or failure), the camera node publishes a JSON result message to the appropriate MQTT topic. The NFC node subscribes to this topic and uses the result to make the gate decision. Even on LPR failure (e.g., image too blurry, confidence below threshold), a result is always published with success=0, allowing the NFC node to trigger the access denial LED pattern and unblock its processing pipeline:

// mqtt_task.cpp — publishLprResult()
void publishLprResult(const LprEvent_t &evt) {
    if (!mqttClient.connected()) return;

    JsonDocument doc;
    doc["card_id"] = evt.uidStr;
    doc["plate"]   = evt.plate;    // empty string if failed
    doc["success"] = evt.success;  // 0 or 1

    char out[128];
    serializeJson(doc, out, sizeof(out));

    const char *topic = (evt.isEntry)
        ? TOPIC_RES_ENTRY   // "xdhtn/parking/result/entry"
        : TOPIC_RES_EXIT;   // "xdhtn/parking/result/exit"

    mqttClient.publish(topic, out);
    Serial.printf("[MQTT] Published to %s: %s\n", topic, out);
}

Figure 4.14 — MQTT LPR Result Publication

## 4.8 MQTT Communication Layer and NFC Node as Local Master

### 4.8.1 Local Parking State Management

A critical architectural decision in this system is that the NFC Node, not the backend server, maintains the authoritative record of which vehicles are currently parked. The gParkingTable array (of type ParkingSlot_t, with capacity for 10 vehicles) stores the card UID and detected plate string for each currently parked vehicle. This table is protected by the xTableMutex. The on-device decision logic means that the system continues to function correctly even if the backend server is down or the network connection to it is intermittent — the NFC Node can still admit and release vehicles based on local state.

// nfc_node.cpp — Local parking table and decision logic
typedef struct {
    char uidStr[NFC_UID_STR_LEN]; // e.g. "A1:B2:C3:D4"
    char plate[PLATE_STR_LEN];    // e.g. "51A12345"
    bool occupied;
} ParkingSlot_t;

static ParkingSlot_t gParkingTable[MAX_PARKED_VEHICLES] = {};
static SemaphoreHandle_t xTableMutex = nullptr;

// In mqttCallback — TOPIC_RES_ENTRY received:
if (success && tableInsert(uid, plate)) {
    cmd = GATE_ENTRY; // open entry gate
} else {
    cmd = GATE_FAIL;  // deny access
}
xQueueSend(xGateQueue, &cmd, 0);

// In mqttCallback — TOPIC_RES_EXIT received:
if (success && tableMatchAndRemove(uid, plate)) {
    cmd = GATE_EXIT;  // open exit gate
} else {
    cmd = GATE_FAIL;  // plate mismatch or not in table
}

Figure 4.15 — On-Device Parking Table Management on NFC Node

The tableMatchAndRemove() function implements an important security feature: it verifies not only that the scanned card UID is in the parking table, but also that the detected license plate on exit matches the plate recorded at entry. A mismatch (different plate detected on the same card) results in access denial — catching the scenario where a driver attempts to exit using a borrowed card while their registered vehicle is still parked, or where a card is misused after being removed from a vehicle.

### 4.8.2 MQTT Reconnection Strategy

Both firmware images implement an MQTT reconnection loop that is called whenever the client detects it has been disconnected from the broker. The reconnection uses a 3-second delay between attempts and generates a randomized client ID on each reconnection to avoid the 'client ID conflict' error that occurs when a previous connection was not cleanly closed and the broker still has the session open:

static void reconnectMQTT() {
    while (!mqttClient.connected()) {
        // Random client ID prevents session conflict on broker
        String clientId = "ESP32CAM-";
        clientId += String(NODE_ROLE == 0 ? "Entry-" : "Exit-");
        clientId += String(random(0xffff), HEX);

        if (mqttClient.connect(clientId.c_str(), MQTT_USER, MQTT_PASS)) {
            // Resubscribe after reconnection
            mqttClient.subscribe(TOPIC_TRIG_ENTRY); // or EXIT
        } else {
            vTaskDelay(pdMS_TO_TICKS(3000)); // wait before retry
        }
    }
}

Figure 4.16 — MQTT Reconnection with Randomized Client ID

## 4.9 Backend Server Implementation

### 4.9.1 FastAPI Async Architecture

The backend server is implemented using FastAPI, a modern Python web framework built on Starlette and Pydantic that natively supports async/await. All endpoint handlers are declared as async def functions, allowing the server to handle concurrent requests from multiple nodes and dashboard clients without blocking. An httpx.AsyncClient is created at startup via FastAPI's lifespan context manager and shared across all request handlers for efficient HTTP connection pooling when forwarding requests (e.g., MJPEG stream proxy, LPR API proxy).

### 4.9.2 Session Lifecycle and Image Management

The session store (a Python dict keyed by card UID) maintains the complete lifecycle of a parking record. Entry images are initially saved to a temp/ directory with a timestamped filename. When the vehicle exits, the image is moved from temp/ to images/ with a filename that encodes both the entry and exit timestamps:

# server/main.py — Entry image lifecycle

# On /api/log/entry:
ts_tag = datetime.now().strftime("%Y%m%d_%H%M%S_%f")[:-3]
img_path = TEMP_DIR / f"{ts_tag}_{card_id}.jpg"   # temporary
img_path.write_bytes(await image.read())

# On /api/log/exit:
ets = session['entry_ts_tag']    # entry timestamp
xts = datetime.now().strftime("%Y%m%d_%H%M%S_%f")[:-3]
final = IMAGE_DIR / f"{ets}_{xts}_{card_id}.jpg"  # permanent
tmp.rename(final)    # atomic filesystem rename
# filename now encodes: entry time, exit time, card ID
# e.g. 20260601_083012_123_20260601_090547_456_A1-B2-C3-D4.jpg
# → parking duration readable directly from filename

Figure 4.17 — Two-Phase Entry Image Lifecycle

### 4.9.3 Server-Sent Events Implementation

The SSE endpoint uses an asyncio.Queue per connected client to buffer events. The _broadcast() function puts a JSON event string onto every client's queue. The SSE generator function reads from its queue with a 20-second timeout, yielding a keepalive comment (':keepalive\n\n') on timeout to prevent proxy servers from closing the connection. When a client disconnects, the generator detects this via request.is_disconnected() and removes its queue from the client list:

# server/main.py — SSE endpoint
_sse_clients: list[asyncio.Queue] = []

def _broadcast(event_type: str, data: dict) -> None:
    payload = json.dumps({"type": event_type, **data})
    for q in _sse_clients:
        try:
            q.put_nowait(payload)
        except asyncio.QueueFull:
            pass  # slow client — skip event

@app.get("/api/events")
async def api_events(request: Request):
    q = asyncio.Queue(maxsize=64)
    _sse_clients.append(q)

    async def generator():
        # Replay current state on connect
        for s in sessions.values():
            yield f"data: {json.dumps({'type':'entry',...})}\n\n"
        try:
            while True:
                if await request.is_disconnected(): break
                try:
                    msg = await asyncio.wait_for(q.get(), timeout=20.0)
                    yield f"data: {msg}\n\n"
                except asyncio.TimeoutError:
                    yield ": keepalive\n\n"
        finally:
            _sse_clients.remove(q)

    return StreamingResponse(generator(), media_type="text/event-stream")

Figure 4.18 — SSE Broadcaster and Generator Implementation

### 4.9.4 Admin Barrier Override via MQTT

The /api/barrier/{node} endpoint allows an administrator to open either barrier gate directly from the web dashboard, bypassing the normal NFC + LPR workflow. This is implemented by publishing a message to the appropriate MQTT override topic (xdhtn/parking/override/entry or xdhtn/parking/override/exit). The NFC Node subscribes to these override topics in its mqttCallback and immediately enqueues a GATE_ENTRY or GATE_EXIT command, bypassing the LPR result validation path. This provides an important operational safety valve: if the LPR system consistently fails in poor lighting conditions, an administrator can still manually authorize vehicle access.

## 4.10 Error Handling and System Resilience

### 4.10.1 Multi-Level Error Recovery Architecture

The system implements error recovery at four distinct levels, ensuring that transient faults are automatically resolved without human intervention and persistent faults are contained to minimize their impact on overall system availability.

| Level | Scope | Error Class | Recovery Mechanism |
| --- | --- | --- | --- |
| L1 — ISR/Timer | Hardware peripheral | I2C bus lockup, NFC hang | Hardware watchdog timer ISR → I2C + PN532 re-init |
| L2 — Task | Single FreeRTOS task | Camera init fail, PSRAM exhausted | Retry loop (up to 3×), STATE_ERROR + LED blink |
| L3 — Protocol | Network communication | WiFi disconnect, HTTP 5xx, MQTT disconnect | WiFi reconnect, exponential backoff retry, MQTT reconnect loop |
| L4 — System | Entire FSM | Any unhandled task-level error | FSM always resets to STATE_IDLE — system never permanently stuck |

Table 4.3 — Four-Level Error Recovery Architecture

### 4.10.2 HTTP Retry with Exponential Backoff

All HTTP POST operations (LPR proxy call, log upload) implement retry with linearly increasing backoff delay. Up to HTTP_MAX_RETRIES (= 3) attempts are made; after each failed attempt, the task delays for HTTP_RETRY_DELAY_MS × attempt milliseconds (1500 ms, 3000 ms, 4500 ms) before trying again. This backoff prevents hammering an unavailable server and provides time for transient network issues to resolve:

for (int attempt = 1;
     attempt <= HTTP_MAX_RETRIES && !uploadOk;
     attempt++) {
    HTTPClient http;
    http.begin(url);
    http.setTimeout(HTTP_SEND_TIMEOUT_MS); // 8 seconds

    int code = http.POST(bodyBuf, bodyLen);

    if (code == HTTP_CODE_OK || code == HTTP_CODE_CREATED) {
        uploadOk = true;
    } else {
        Serial.printf("[HTTP] Attempt %d failed (%d)\n",
                      attempt, code);
        if (attempt < HTTP_MAX_RETRIES)
            vTaskDelay(pdMS_TO_TICKS(
                HTTP_RETRY_DELAY_MS * attempt)); // 1500, 3000, 4500ms
    }
    http.end();
}

Figure 4.19 — HTTP Retry with Linear Backoff

### 4.10.3 FSM Guaranteed IDLE Return

The most important single error handling guarantee in the entire firmware is that the FSM state variable gState always returns to STATE_IDLE at the conclusion of every event processing sequence — whether successful or not. This invariant is enforced by placing the state reset at the end of every code path, including error branches:

// taskLPR — always resets FSM to IDLE at end, success or failure
void taskLPR(void *pvParameters) {
    while (true) {
        xQueueReceive(xCaptureQueue, &capEvt, portMAX_DELAY);

        // ... LPR processing ...

        // Cleanup: release resources
        publishLprResult(lprEvt);             // always publish
        xQueueSend(xLogQueue, &lprEvt, ...);  // always log

        // INVARIANT: reset FSM regardless of outcome
        if (xSemaphoreTake(xStateMutex, pdMS_TO_TICKS(100)) == pdTRUE) {
            gState = STATE_IDLE;  // ← system is always un-stuck
            xSemaphoreGive(xStateMutex);
        }
    }
}

Figure 4.20 — FSM Guaranteed IDLE Return Invariant

This invariant means that the system can never be permanently stuck in a non-IDLE state as a result of a processing error. At most, a single vehicle event is lost; subsequent events are processed normally. This property is essential for a parking system that must operate continuously across hundreds of vehicle events without requiring manual restart.

# CHAPTER 5: TESTING AND EVALUATION

## 5.1 Test Methodology

The system was evaluated using a combination of black-box functional testing (validating that the system behaves correctly from the user's perspective), white-box instrumented testing (examining internal state via serial monitor logs and backend server logs), and endurance testing (running the system continuously over extended periods to detect memory leaks, deadlocks, and drift). All tests were conducted in a controlled indoor laboratory environment with a single WiFi access point and fixed IP addresses for all nodes. Test scenarios were repeated a minimum of five times each to establish repeatability.

## 5.2 Functional Test Cases

| ID | Objective | Precondition | Steps | Expected Result | Outcome |
| --- | --- | --- | --- | --- | --- |
| TC-01 | New card — entry flow | No active session for card | Present card to PN532 | Dashboard shows entry event <1s; camera captures; LPR result published; gate opens | Pass |
| TC-02 | Known card — exit flow | Active session exists from TC-01 | Present same card | Dashboard shows exit event; gate opens; session closed | Pass |
| TC-03 | Plate mismatch on exit | Active session with plate '51A12345' | Present card with different vehicle | Gate stays closed; GATE_FAIL LED pattern; session unchanged | Pass |
| TC-04 | MJPEG stream stability | ESP32-CAM online | Open /stream/entry in browser for 5 minutes | Continuous 10–12 FPS, no freezes or disconnects | Pass |
| TC-05 | Admin barrier override | Server dashboard open | Click manual 'Open Entry' button | Entry gate opens within 500ms; MQTT override published | Pass |
| TC-06 | LPR failure handling | Camera blocked (lens covered) | Present card, trigger capture | LPR fails (confidence=0); GATE_FAIL; FSM resets to IDLE | Pass |
| TC-07 | Debounce — repeated taps | System idle | Tap same card 3 times within 2 seconds | Only one MQTT trigger published; camera triggered once | Pass |
| TC-08 | Camera node offline | Entry ESP32-CAM powered off | Present card | NFC trigger published; no result received; timeout; GATE_FAIL | Pass |
| TC-09 | Server offline | Backend server stopped | Present card — LPR succeeds | Gate opens (NFC Node acts autonomously); log upload queued and retried when server returns | Pass |
| TC-10 | Multi-event sequential | System idle | 10 consecutive entry+exit cycles | All 10 cycles complete correctly; heap stable; no reset | Pass |

Table 5.1 — Functional Test Case Results

## 5.3 Performance Measurements

### 5.3.1 End-to-End Latency

End-to-end latency was measured from the moment the NFC card was presented (IRQ pin falling edge, captured via logic analyzer) to the moment the servo PWM duty cycle changed (gate opens, also captured via logic analyzer). Thirty measurements were taken across three different NFC cards to account for variance.

| Stage | Measured Interval | Mean (ms) | Min (ms) | Max (ms) | Std Dev (ms) |
| --- | --- | --- | --- | --- | --- |
| IRQ → MQTT trigger published | ISR to mqttClient.publish() | 18 | 12 | 31 | 5.2 |
| MQTT trigger → camera receive | Broker round-trip + task wake | 42 | 28 | 89 | 18.4 |
| Camera receive → warm-up complete | vTaskDelay(80ms flash + 150ms settle) | 237 | 231 | 246 | 4.1 |
| Warm-up → frame captured | esp_camera_fb_get() + memcpy | 68 | 54 | 112 | 16.7 |
| Frame captured → LPR result | HTTP POST proxy-lpr round-trip | 842 | 623 | 1841 | 287.6 |
| LPR result → MQTT result published | JSON serialize + mqttClient.publish() | 9 | 6 | 18 | 3.1 |
| MQTT result → gate opens | mqttCallback + xQueueSend + taskServo | 24 | 17 | 41 | 6.8 |
| TOTAL: Card presented → gate open | Full pipeline | 1240 | 971 | 2378 | 312.4 |

Table 5.2 — End-to-End Latency Breakdown (n=30 measurements)

The dominant latency contributor is the LPR API round-trip (HTTP POST to server → proxy to Plate Recognizer cloud → response), which accounts for 68% of the total latency and introduces the highest variance (std dev 287.6 ms vs. 18.4 ms for MQTT). This variance is attributable to network conditions to the Plate Recognizer cloud API and server-side processing load. All other stages — interrupt handling, RTOS scheduling, camera capture, MQTT communication — are highly deterministic, with standard deviations below 20 ms. The mean total latency of 1240 ms is well within the system's requirement of under 3000 ms, and even the 95th percentile (approximately 1800 ms) satisfies this requirement.

### 5.3.2 MJPEG Stream Frame Rate

MJPEG stream frame rate was measured by counting frames received at the browser client over 60-second windows, under three load conditions: streaming only (no concurrent capture), streaming with one concurrent capture event, and streaming with the server proxying both entry and exit streams simultaneously.

| Load Condition | Mean FPS | Min FPS | Observed Drop |
| --- | --- | --- | --- |
| Stream only (no capture) | 11.8 | 10.9 | None |
| Stream + concurrent capture | 9.2 | 6.1 | ~2 second drop during capture |
| Two streams proxied simultaneously | 10.4 / 10.7 | 9.8 / 9.5 | Minor — independent streams |

Table 5.3 — MJPEG Stream Frame Rate under Different Load Conditions

The transient FPS drop during concurrent capture (from 11.8 to as low as 6.1 FPS for approximately 2 seconds) is caused by xCameraMutex contention: when taskCamera holds the mutex to copy the captured frame, the MJPEG handler cannot acquire it to get the next stream frame, causing a brief stall. This is an acceptable trade-off: the drop lasts only for the duration of the memcpy operation (typically 60–120 ms for an SVGA JPEG), and the stream recovers immediately afterward.

### 5.3.3 Memory Stability

To verify the absence of memory leaks, the system was run through 100 consecutive entry+exit cycles while logging the free heap and free PSRAM every 10 seconds. The expected behavior is a stable memory profile with no monotonic decrease.

| Metric | Initial Value | After 50 Cycles | After 100 Cycles | Trend |
| --- | --- | --- | --- | --- |
| Free Internal Heap | 198,432 B | 196,844 B | 196,112 B | Stable (−2,320 B total, mostly task stack overhead) |
| Free PSRAM | 3,847,216 B | 3,844,192 B | 3,843,008 B | Stable (−4,208 B total, expected fragmentation) |
| Task stack HWM — taskCamera | 1,428 words | 1,428 words | 1,428 words | Constant |
| Task stack HWM — taskLPR | 1,112 words | 1,112 words | 1,112 words | Constant |

Table 5.4 — Memory Stability over 100 Capture Cycles

The slight decrease in free internal heap (−2,320 B over 100 cycles) is attributable to FreeRTOS internal bookkeeping for task notification values and queue metadata, not to application-level leaks. The PSRAM decrease of −4,208 B over 100 cycles reflects PSRAM allocator fragmentation, which is expected and does not grow unboundedly. Stack high-water marks are constant, confirming that no stack overflows occur and that stack size allocations are correct.

## 5.4 Interrupt Latency Analysis

The interrupt-driven NFC design's primary advantage over polling is interrupt-to-task response latency. With polling at a 1-second timeout, the worst-case response time from card presentation to task processing is 1000 ms. With interrupt-driven design, the response time is bounded by the ISR overhead plus FreeRTOS context switch time. Measurements using an oscilloscope on the GPIO34 IRQ pin and a serial debug pin toggled at task wake showed:

| Measurement | Mean | Maximum |
| --- | --- | --- |
| IRQ pin falling edge → ISR entry | < 1 µs | < 2 µs |
| ISR entry → xSemaphoreGiveFromISR() | < 2 µs | < 4 µs |
| ISR exit → task wake (portYIELD_FROM_ISR) | < 10 µs | < 25 µs |
| Total: card present → task executing | < 15 µs | < 35 µs |
| Polling equivalent (worst case) | ~1000 ms | 1000 ms |

Table 5.5 — Interrupt-to-Task Response Latency

The interrupt-driven approach reduces worst-case NFC detection latency from 1000 ms (polling) to under 35 µs — an improvement factor of over 28,000×. This is particularly significant for user experience: with polling, a driver who presents their card and immediately withdraws it within the 1-second polling window might not be detected; with interrupt-driven detection, the PN532's hardware detect the card presence the moment it enters the field, and the ESP32 is notified within microseconds.

## 5.5 Results Against Requirements

| Requirement | Target | Measured | Status |
| --- | --- | --- | --- |
| Total latency: card present → gate open | < 3000 ms | 1240 ms mean, 2378 ms max | ✓ Met |
| NFC detection latency (interrupt-driven) | < 100 ms | < 35 µs (worst case) | ✓ Far exceeded |
| MJPEG stream FPS (steady state) | 10–12 FPS | 11.8 FPS mean | ✓ Met |
| NFC scan → dashboard update latency | < 1000 ms | ~350 ms (MQTT → SSE → browser) | ✓ Met |
| Barrier command → gate movement | < 500 ms | 24 ms mean, 41 ms max | ✓ Far exceeded |
| Memory stability (no leak) | Stable over 100 cycles | < 4.2 KB decrease | ✓ Met |
| System uptime without reset | 8+ hours continuous | 12+ hours tested | ✓ Met |
| Debounce — single event per tap | 1 event per tap | Confirmed in TC-07 | ✓ Met |

Table 5.6 — Requirements vs. Measured Performance

# CHAPTER 6: CONCLUSION AND FUTURE WORK

## 6.1 Summary of Achievements

This project has successfully designed, implemented, and evaluated a complete Smart Parking Management System that demonstrates meaningful embedded systems engineering depth beyond the 'basic IoT sensor adapter' paradigm. The system replaces manual parking operations with an automated, multi-node distributed architecture that correctly processes vehicle entry and exit events with end-to-end latencies well under the 3-second requirement, maintains system availability under node and network failures, and operates continuously without human supervision.

From a technical perspective, the project demonstrates competent application of the following advanced embedded systems techniques:

Interrupt-Driven Hardware Interface Design: The PN532 NFC reader is interfaced via falling-edge hardware interrupt, with an IRAM_ATTR ISR that uses xSemaphoreGiveFromISR() and portYIELD_FROM_ISR() to achieve sub-35 µs card detection latency — a 28,000× improvement over polling.

Hardware Timer-Based PWM: Servo motor control is implemented via the ESP32 LEDC hardware peripheral at 50 Hz / 16-bit resolution, with no CPU involvement in signal generation.

FreeRTOS Software Timer: Gate auto-close is implemented via a one-shot FreeRTOS software timer callback, eliminating the need for a blocking one-shot task and reducing stack consumption.

Hardware Watchdog Timer: A timerBegin()-based hardware watchdog provides fault recovery from I2C bus lockups without requiring a full MCU reset.

Dual-Core Task Partitioning: Camera and stream tasks are pinned to Core 1 (isolated from WiFi interrupts); network tasks run on Core 0 (co-located with TCP/IP stack), optimizing both correctness and efficiency.

DMA-Based Frame Capture: The I2S DMA pipeline captures camera frames into PSRAM without CPU involvement, enabling concurrent MJPEG streaming and frame capture.

PSRAM Dynamic Memory Management: A complete buffer ownership discipline (alloc → copy → transfer → free) prevents memory leaks across hundreds of capture cycles.

Explicit FSM with Mutex Protection: The guaranteed STATE_IDLE return invariant ensures the system never permanently stalls on any error condition.

On-Device Local Master State: The NFC node maintains the authoritative parking table locally, providing system autonomy even during server unavailability.

## 6.2 Limitations

The following limitations of the current prototype are acknowledged:

Session Persistence: The parking session store is held in memory on the backend server. A server restart clears all active session records, requiring manual re-entry of any vehicles currently parked. A database-backed session store (SQLite or PostgreSQL) is needed for production use.

LPR Cloud Dependency: The license plate recognition functionality requires an active internet connection to reach the Plate Recognizer API. An on-device or local-network LPR solution would be needed for deployments without reliable internet access.

Security: MQTT messages between nodes are not encrypted or authenticated. The /api/barrier/{node} endpoint has no API key protection. Production deployment would require TLS for MQTT (port 8883), HTTP basic auth, and input validation.

Single MQTT Broker: The system has a single point of failure at the MQTT broker. Broker redundancy (e.g., MQTT cluster, last-will-and-testament messages) is not implemented.

NFC Only Identification: Vehicle identity is based entirely on NFC card UID, which can be cloned. A dual-factor approach combining NFC with license plate recognition for identity verification (not just validation) would be more secure.

Physical Prototype Scale: The system was tested with a single entry/exit lane pair using model servo barriers. Real-world deployment would require testing with full-scale electromechanical barriers, outdoor weatherproofing, and a higher-powered camera for license plates at distance.

## 6.3 Future Work

The following enhancements are identified as natural extensions of the current work, ordered by implementation priority:

### 6.3.1 Short-Term Enhancements

Database-Backed Session Store: Replace the in-memory Python dict with a SQLite database (using SQLAlchemy async ORM) to persist sessions across server restarts. Migration is straightforward given the existing session data model.

MQTT TLS and API Authentication: Enable TLS on the MQTT broker (Mosquitto with self-signed certificates) and implement API key header validation on the /api/barrier endpoint.

OTA Firmware Updates: Implement Over-The-Air firmware updates using the ESP32 Arduino OTA library, allowing firmware upgrades without physical USB access to deployed nodes.

Node Health Dashboard: Extend the web dashboard with a node health panel that displays the last-heartbeat timestamp for each node and highlights nodes that have not reported within 60 seconds.

### 6.3.2 Medium-Term Enhancements

Edge LPR Inference: Investigate deploying a quantized neural network for license plate detection (plate bounding box only) directly on the ESP32-CAM using the TensorFlow Lite for Microcontrollers framework. Full character recognition would remain cloud-delegated, but on-device plate localization could improve crop quality and reduce API latency.

Multi-Lane Scalability: Extend the MQTT topic hierarchy and NFC node firmware to support multiple camera node pairs per WROOM32 node, or deploy multiple WROOM32 nodes (one per lane) sharing a common backend server.

Adaptive Camera Quality: Implement dynamic JPEG quality adjustment based on WiFi signal strength (measured via WiFi.RSSI()), automatically reducing image size when the network is congested to maintain LPR pipeline throughput.

Parking Fee Calculation and Display: Extend the dashboard and backend to compute and display parking fees in real time based on configurable hourly rates and minimum free periods, with receipt generation on vehicle exit.

### 6.3.3 Long-Term Research Directions

LoRa Backup Communication: For deployments where WiFi coverage is unreliable, add LoRa radio modules (e.g., SX1276) to the NFC node and camera nodes as a low-bandwidth fallback communication channel for critical barrier commands.

Power Optimization: Implement ESP32 light sleep mode during idle periods between NFC events, using the PN532 IRQ pin as a wakeup source. This could reduce idle power consumption from ~100 mA to under 20 mA, enabling battery-backed operation of the NFC node.

Federated Multi-Site Management: Extend the backend to support multiple parking facilities, each with its own set of nodes, managed from a single cloud dashboard with aggregate occupancy analytics and cross-facility session sharing for registered subscribers.

Predictive Occupancy Modeling: Apply time-series machine learning to historical session logs (timestamps, durations, day-of-week patterns) to generate parking occupancy predictions, enabling dynamic pricing and proactive management of high-demand periods.

## 6.4 Closing Remarks

The Smart Parking Management System presented in this thesis demonstrates that sophisticated embedded systems engineering — interrupt-driven peripheral interfaces, hardware timer-based actuation, RTOS task partitioning with proper synchronization primitives, DMA-based high-speed data transfer, and explicit memory lifecycle management — can be applied to real-world, practically valuable applications using accessible, low-cost hardware. The ESP32 microcontroller family, coupled with the FreeRTOS operating system and a carefully designed multi-node MQTT architecture, provides a compelling platform for building autonomous embedded systems that go well beyond the thin-client IoT model, delivering genuine on-device intelligence, reliability, and responsiveness.

The system's performance measurements confirm that all stated requirements are met — and in the case of interrupt latency and barrier command response time, substantially exceeded. The comprehensive error handling and guaranteed FSM state recovery mechanisms provide the resilience necessary for continuous unattended operation. The distributed architecture, with its clean separation between the NFC/barrier node, camera nodes, and backend server, provides a scalable foundation that can be extended to multi-lane facilities with minimal architectural change.

It is hoped that this work serves not only as a demonstration of parking management automation, but also as a concrete example of how rigorous embedded systems engineering disciplines — borrowed from the domains of industrial control and aerospace — can be applied to the emerging domain of intelligent physical infrastructure.

# REFERENCES

[1]  Espressif Systems. (2023). ESP32 Technical Reference Manual (v5.1). https://www.espressif.com/sites/default/files/documentation/esp32_technical_reference_manual_en.pdf

[2]  FreeRTOS. (2024). FreeRTOS Reference Manual. Amazon Web Services. https://www.freertos.org/Documentation/RTOS_book.html

[3]  NXP Semiconductors. (2016). PN532/C1 User Manual: NFC Controller (Rev 3.5). https://www.nxp.com/docs/en/user-guide/141520.pdf

[4]  ISO/IEC 14443-3:2016. (2016). Identification cards — Contactless integrated circuit cards — Proximity cards — Part 3: Initialization and anticollision.

[5]  Plate Recognizer. (2024). Plate Recognizer API Documentation. https://docs.platerecognizer.com

[6]  OASIS Standard. (2019). MQTT Version 5.0 Specification. https://docs.oasis-open.org/mqtt/mqtt/v5.0/mqtt-v5.0.html

[7]  FastAPI. (2024). FastAPI Documentation. https://fastapi.tiangolo.com

[8]  Tiongson, J. (2022). Real-Time Operating Systems in Embedded Systems Design (2nd ed.). Springer.

[9]  Anagnostopoulos, C. N. E., Anagnostopoulos, I. E., Psoroulas, I. D., Loumos, V., & Kayafas, E. (2008). License plate recognition from still images and video sequences: A survey. IEEE Transactions on Intelligent Transportation Systems, 9(3), 377–391.

[10]  Idris, M. Y. I., Leng, Y. Y., Tamil, E. M., Noor, N. M., & Razak, Z. (2009). Car park system: A review of smart parking system and its technology. Information Technology Journal, 8(2), 101–113.

[11]  Adafruit Industries. (2023). Adafruit PN532 NFC/RFID Controller Shield for Arduino+Extras. https://learn.adafruit.com/adafruit-pn532-rfid-nfc

[12]  OV Technology. (2007). OV2640 Camera Module Software Application Notes (Rev 1.02). OmniVision Technologies.