Link merge bài giảng PDF: computer-architecture-full-teacher-Dau.pdf

Question 1: The computer memory system is usually organized in hierarchical levels and the memory hierarchy roles are to _____

a. Improve the system efficiency and reduce the sale costs

b. Improve the system performance and reduce the manufacturing costs

c. Improve the system performance and reduce the sale costs

d. Improve the system efficiency and reduce the manufacturing costs

Explanation: Cấu trúc phân cấp (hierarchical level) trong hệ thống nhớ (memory system) có hai vai trò chính: (1) tăng hiệu năng hệ thống (improve the system performance) thông qua việc giảm thời gian truy cập các ô nhớ và (2) giảm giá thành sản xuất (reducing the manufacturing costs)

(Trang 41 giáo trình chương 4, trang 170 slide)

Question 2: What are the characteristics of ROM memory?

a. Volatile memory, magnetic memory, read only memory

b. Volatile memory, semiconductor memory, read only memory

c. Non-volatile memory, semiconductor memory, random access memory

d. Non-volatile memory, semiconductor memory, read only memory

Explanation: Bộ nhớ ROM có các tính chất (characteristic) sau: (1) là bộ nhớ chỉ đọc (read only memory), (2) thuộc loại bộ nhớ bán dẫn (semiconductor memory) và (3) bộ nhớ ổn định (non-volatile memory)

(Trang 42 giáo trình chương 4, trang 174 slide)

Question 3: What is the meaning of the instruction “ADD A, B, C”, where A, B, C are memory locations?

a. M[B] ⟵ M[A]+M[C]

b. M[C] ⟵ M[A]+M[B]

c. M[A] ⟵ M[A] + M[B] + M[C]

d. M[A] ⟵ M[B] + M[C]

Explanation: Công thức lệnh cộng: “ADD A, B, C” tức M[A] ⟵ M[B] + M[C], tức cộng nội dung 2 ô nhớ B và C và lưu kết quả vào ô nhớ A

(Trang 31 giáo trình chương 3)

Question 4: What is the correct name of NAS storage system?

a. Net Attacked Storage

b. Net Attached Storage

c. Network Attached Storage

d. Network Attacked Storage

Explanation: NAS – Network Attached Storage, hệ thống lưu trữ gắn vào mạng

(Trang 72 giáo trình chương 5, trang 291 slide)

Question 5: What is the data transfer speed of the third SATA interface generation?

a. 5 Gb/s

b. 6 Gb/s

c. 4 Gb/s

d. 8 Gb/s

Explanation: Tốc độ truyền dữ liệu của SATA thế hệ 1, thế hệ 2, và thế hệ 3 lần lượt là 1.5GB/s, 3GB/s và 6GB/s

(Trang 64 giáo trình chương 5, trang 246 slide)

Question 6: What is an input peripheral device for computers?

a. Monitor

b. CD-RW drive

c. Printer

d. Plotter

Explanation: Màn hình máy tính (Monitor), máy in (printer) và máy vẽ (plotter) đều là thiết bị ngoại vi ra (output peripheral device). Còn CD-RW drive đóng vai trò là cả thiết bị ngoại vi vào và ra

(Google)

Question 7: What are data transfer speeds of v1.x, v2.0, v3.0 PCIe 16 lanes?

a. 4GB/s, 8GB/s, 16GB/s

b. 4GB/s, 8GB/s, 12GB/s

c. 8GB/s, 12GB/s, 16GB/s

d. 8GB/s, 16GB/s, 32GB/s

Explanation: Tốc độ truyền dữ liệu (data transfer speed) của PCIe tương ứng với phiên bản 1.x, 2.0, 3.0 là 250MB/s, 500MB/s, 1GB/s đối với một luồng (lane). Mà tốc độ truyền dữ liệu phụ thuộc vào số luồng sử dụng, vậy nên 16 luồng thì bằng tốc độ ban đầu nhân 16, tương ứng với 4GB/s, 8GB/s, và 16GB/s

(Trang 81 giáo trình chương 6, slide trang 321)

Question 8: What are main components of a lazer printer?

a. Photo drum, Laser light source, Rotating mirror and modulator, Toner cartridge

b. Photosensitive drum, Laser light source, Rotating mirror and laser modulator, Toner cartridge

c. Photosensitive drum, Laser light source, Rotating and laser modulator, Toner box

d. Photo drum, Laser light source, Rotating mirror and modulator, Toner box

Explanation: Máy in lazer (Lazer printer) gồm các thành phần chính sau: trống cảm quang (photosensitive drum), nguồn sáng laser (laser light source), gương quay (rotating mirror), bộ điều chế tia (laser modulator), hộp mực (toner cartridge), trống sấy nóng (drying drum), khay giấy (paper tray), điện cực nạp cho trống (electrode charger for drum), điện cực nạp cho giấy (electrode charger for paper)

(Trang 90 giáo trình chương 6, slide trang 367)

Question 9: SRAM is fast, but expensive, so it is usually used for small-in-size and fast memory elements, such as _____

a. registers, cache.

b. cache, RAM.

c. cache, ROM.

d. ROM, RAM.

Explanation: SRAM nhanh và đắt hơn DRAM và thường được sử dụng cho bộ nhớ cache và thanh ghi bên trong CPU, còn DRAM thì thường dùng cho bộ nhớ chính

(Wikipedia)

Question 10: The Control Unit (CU) decodes the instruction and generates _____ for other units, such as ALU execute the instruction.

a. command

b. pulse

c. execution signal

d. request

Explanation: Bộ điều khiển CU lấy lệnh, giải mã lệnh và sinh các tín hiệu điều khiển (control signal, execution signal), yêu cầu các bộ phận khác như ALU thực hiện lệnh

(Trang 19 giáo trình, slide trang 19 chương 2)

Question 11: In “Look Aside” architecture, cache and memory are connected to _____ and they "see" the CPU bus cycle _____

a. internal bus / at the different time.

b. system bus/ at the same time.

c. system bus / at the different time.

d. internal bus / at the same time.

Explanation: Trong kiến trúc Look Aside, cache và bộ nhớ chính cùng kết nối vào bus hệ thống (system bus), và cả hai đều “thấy” chu kỳ bus của CPU tại cùng một thời điểm

(Trang 48 giáo trình, slide trang 31 chương 4)

Question 12: Instruction execution is divided into phases or stages. One instruction can be executed in 5 stages, including _____

a. IF, IR, EX, MEM, WR

b. IS, ID, EX, MEN, WB

c. IF, ID, EX, MEM, WB

d. IC, ID, EX, MEM, WR

Explanation: Việc thực hiện lệnh được chia thành 5 giai đoạn: Instruction Fetch (IF), Instruction Decode (ID), Execute (EX), Memory Access (MEM), Write Back (WB)

(Trang 33 giáo trình, slide trang 4 chương 3)

Question 13: What is the correct full name of RAID storage system?

a. Redundance Array of Independence Disks

b. Redundant Array of Independent Disks

c. Redundant Array of Individual Disks

d. Reused Array of Independent Disks

Explanation: RAID - Redundant Array of Independent Disks, công nghệ lưu trữ RAID, tạo thành từ một mảng liên kết các đĩa cứng vật lý

(Trang 7 giáo trình, trang 69 giáo trình, slide trang 4 chương 3)

Question 14: What are common computer system buses?

a. ISA, EISA, PCI, AGP, PCIe

b. ISA, TISA, PCI, PGP, PCIe

c. ISA, EISS, PCIg, AGP, PCIe

d. ISS, EISA, PCI, AGP, PCIex

Explanation: Việc thực hiện lệnh được chia thành 5 giai đoạn: Instruction Fetch (IF), Instruction Decode (ID), Execute (EX), Memory Access (MEM), Write Back (WB)

(Trang 76-81 giáo trình chương 6, slide trang 3 chương 5)

Question 15: In computer functional block diagram, there are CPU, system bus, _____

a. internal memory and peripheral devices.

b. external memory and input devices.

c. external memory and peripheral devices.

d. internal memory and output devices.

Explanation: Sơ đồ khối chức năng của hệ thống máy tính (computer functional block diagram) bao gồm (1) CPU, (2) bộ nhớ trong (internal memory), (3) các thiết bị ngoại vi (peripheral devices) và (4) hệ thống bus (bus system)

(Trang 8-9 giáo trình chương 2)

Question 16: What is the addressing mode of the instruction "LOAD R1, #1000", where R1 is a CPU register?

a. Direct

b. Indirect

c. Relative

d. Immediate

Explanation: Trong chế độ địa chỉ tức thì (Immediate addressing mode), toán hạng nguồn là một giá trị hằng, toán hạng đích là thanh ghi hoặc địa chỉ ô nhớ. Ở đây, R1 là thanh ghi, giá trị hằng là 1000, và ta tải giá trị 1000 vào thanh ghi R1

(Trang 27 giáo trình, slide trang 14 chương 3)

Question 17: In cache temporal locality, if a memory location is accessed, the probability that it is _____ is high.

a. accessed in near future

b. not accessed in near future

c. not accessed again in near future

d. accessed again in near future

Explanation: Nguyên lý lân cận về thời gian (Temporal locality) được phát biểu như sau: Nếu một ô nhớ đang được truy cập thì xác suất nó được truy cập lại trong tương lai gần là rất cao

(Trang 46 giáo trình, slide trang 28 chương 4)

Question 18: The components of memory system include _____

a. Registers, ROM, RAM, Cache, External storages.

b. Registers, ROM, REM, Cache, External Disks.

c. Registers, ROM, RAM, Cache, External HDDs.

d. Registers, ROM, RAM, Cache, SSDs and HDDs.

Explanation: Hệ thống nhớ (memory system) bao gồm thanh ghi (register), bộ nhớ cache, bộ nhớ chính (gồm ROM, RAM), bộ nhớ ngoài (external storage)

(Trang 41 giáo trình, slide trang 5-6 chương 4)

Question 19: What are external storages of computer systems?

a. FFD, HDD, SSD, DVD, RAID, NAS, LAN

b. FDD, HSD, SDD, DVD, RAD, NAS, SAN

c. FFD, HDD, SSD, DVD, RAD, NAT, SAN

d. FDD, HDD, SSD, DVD, RAID, NAS, SAN

Explanation: Bộ nhớ ngoài trong hệ thống nhớ bao gồm: Ổ đĩa mềm (FDD), HDD và SSD, đĩa quang (DVD), RAID, thiết bị gắn mạng (NAS), mạng lưu trữ (NAS)

(Trang 60-73 giáo trình chương 5, slide trang 2 chương 4 phần 6)

Question 20: What are common computer communication ports?

a. PS/1, COM, LPTA, USB, HDMI

b. PS/1, CON, LPT, USB, HDMI

c. PS/2, COM, LPT, USB, HDMI

d. PS/2, COM, LPA, USB, HDMM

Explanation: Các cổng giao tiếp (communication port) thông dụng: PS/2 kết nối chuột và bàn phím, COM, LPT, USB, HDMI

(Trang 83 giáo trình chương 6)

Question 21: The numbers of minimum required HDDs of RAID 0, RAID 1 and RAID 10 are_____

a. 2, 2 and 4.

b. 2, 4 and 4.

c. 2, 2 and 6.

d. 4, 2 and 6

Explanation: RAID 0 và RAID 1 cần tối thiểu 2 ổ đĩa vật lý, RAID 10 cần tối thiểu 4 ổ đĩa

(Trang 70-71 giáo trình chương 5, slide trang 57-59-61 chương 4 phần 6)

Question 22: What is the correct name of PCI bus?

a. Peripheral Component Interconnect

b. Peripher Component Interconnect

c. Peripher Component Interface

d. Peripheral Component Interface

Explanation: PCI – Peripheral Component Interconnect

(Trang 7 giáo trình, trang 77 chương 6, slide trang 11 chương 5 phần 7)

Question 23: A physical HDD can be divided into several parts for better use and management. Each part is called _____

a. a partition.

b. a logical drive.

c. an area.

d. a section.

Explanation: Một đĩa cứng vật lý có thể được chia thành nhiều phần, mỗi phần được gọi là phân đoạn hoặc phân khu (partition)

(Trang 64 giáo trình chương 5, slide trang 22 chương 4 phần 6)

Question 24: Accumulator (A) register is also used in data exchange with _____ devices.

a. input and output

b. internal

c. computer

d. external

Explanation: Thanh ghi tích lũy A có những chức năng sau: (1) lưu toán hạng vào, (2) chứa kết quả ra và (3) dùng trong các lệnh trao đổi dữ liệu với các thiết bị vào ra (input and output devices)

(Trang 19 giáo trình, slide trang 7 chương 2)

Question 25: Which are correct names of file systems?

a. FAT, NTFS, Ext2, Ext4

b. FTA, NTFS, Ext2, Etx4

c. FAT, NTFS, Ext2, etx4

d. FTA, TFS, Ext2, Ext4

Explanation: Hệ thống file (file system) phổ biến: FAT, NTFS, Ext2, Ext3, Ext4, MFS

(Trang 65 giáo trình chương 5, slide trang 24 chương 4 phần 6)

Question 26: Internal memory is used to store instructions and data for CPU to process. Internal memory consists of _____

a. ROM and SAM

b. RON and SAM

c. ROM and RAM

d. RON and RAM

Explanation: Bộ nhớ trong (internal memory) gồm hai loại: bộ nhớ chỉ đọc (ROM) và bộ nhớ truy cập ngẫu nhiên (RAM)

(Trang 9 giáo trình chương 1)

Question 27: CPU instruction is a _____ which implements a single pre-defined operation of a processor.

a. binary code

b. binary word

c. hexa word

d. machine code

Explanation: Lệnh máy tính (instruction) là một từ nhị phân (binary word) được gán một nhiệm vụ cụ thể

(Trang 24 giáo trình, slide trang 3 chương 3)

Question 28: What is correct statement about MAR and MBR registers?

a. MAR is faster than MBR.

b. MAR and MBR are interfaces between the CPU and system bus.

c. MAR is slower than MBR.

d. MAR is the input and MBR is the output of the CPU.

Explanation: Thanh ghi MAR là giao diện giữa CPU và bus địa chỉ, còn thanh ghi MBR là giao diện giữa CPU và bus dữ liệu. Vậy cả MAR và MBR đều là giao diện giữa CPU và hệ thống bus

(Trang 20 giáo trình, slide trang 16 chương 2)

Question 29: One type of data conflicts is Read After Write (RAW). A possible solution for RAW conflict is to _____

a. insert LO-OP instructions into the position between 2 instructions that have RAW.

b. re-write the program.

c. insert NO-OP instructions into the position between 2 instructions that have RAW.

d. re-order instructions of the program.

Explanation: Một trong những giai pháp cho vấn đề tranh chấp dữ liệu kiểu RAW là: chèn them các lệnh NO-OP vào giữa các lệnh gây ra tranh chấp RAW

(Trang 35 giáo trình chương 3, slide trang 17 chương 3 phần 4)

Question 30: What is the addressing mode of the instruction "MOVE (R1), 1000", where R1 is a CPU register?

a. Indexed

b. Relative

c. Direct

d. Indirect

﻿Explanation: Trong chế độ địa chỉ gián tiếp (indirect), một thanh ghi hoặc ô nhớ được dùng để lưu địa chỉ của một ô nhớ khác làm toán hạng. Toán hạng còn lại có thể là một hằng, một thanh ghi hoặc địa chỉ ô nhớ. Ở đây, 1000 là hằng, (R1) là ô nhớ có địa chỉ lưu trong thanh ghi R1, và ta nạp 1000 vào ô nhớ đó

(Trang 28 giáo trình chương 3, slide trang 17 chương 3 phần 3)

Question 31: Two major types of RAM memories are _____

a. SAM, DRAM

b. SRAM, DRAM

c. SRAM, SDRAM

d. RAM, DDR RAM

Explanation: Có 2 loại RAM cơ bản: RAM tính (Static RAM – SRAM) và RAM động (Dynamic RAM – DRAM)

(Trang 43 giáo trình chương 4, slide trang 15 chương 4 phần 5)

Question 32: SCSI and SAS interfaces are usually used in _____ computers

a. laptop

b. desktop

c. server

d. client

Explanation: SCSI là tập các chuẩn về kết nối vật lý và truyền dữ liệu giữa máy tính và thiết bị ngoại vi, thường được sử dụng trong các máy chủ

(Trang 64 giáo trình chương 5, slide trang 19 chương 4 phần 6)

Question 33: What is NOT a component of the instruction set architecture?

a. Address format

b. Processor registers

c. Memory Addressing modes

d. The instruction set

Explanation: Kiến trúc tập lệnh (Instruction set architecture) bao gồm: tập lệnh (instruction set), các chế độ địa chỉ (memory addressing mode), thanh ghi (register), khuôn dạng địa chỉ và dữ liệu (address and data format). Bên trên chỉ có khuôn dạng địa chỉ thì chưa đủ

(Trang 8 giáo trình, slide trang 7 chương 1)

Question 34: Given instruction: “ADD R2, R3, R4 / CMP R1, 0 / JNE somewhere” and each instruction is executed in 2 stages of IF and EX. How many NO-OP instructions are needed to be inserted after the instruction “JNE somewhere” to solve the branching conflict?

a. 0

b. 1

c. 3

d. 2

Explanation: Không biết. Giáo trình bảo thế

(Trang 37 giáo trình chương 3, slide trang 29 chương 3 phần 4)

Question 35: When a program code is loaded into internal memory, the address of the first instruction of the program is set onto _____ register

a. MAR

b. PC

c. FR

d. IR

Explanation: Sau khi mã chương trình được nạp vào bộ nhớ trong, địa chỉ của ô nhớ chứa lệnh đầu tiên của chương trình được nạp vào bộ đếm chương trình PC (thanh ghi PC)

(Trang 19 giáo trình chương 3, slide trang 4 chương 3 phần 3)

Question 36: PCI bus uses parallel communication type. What is the communication type of PCIe bus?

a. Parallel

b. Point-to-Point

c. Serial (Không chắc)

d. Integrated

(Trang 325)

Question 37: What are the semiconductor memories?

a. ROM, RAM, HDD

b. ROM, RAM, SSD

c. ROM, FDD, SSD

d. DVD, RAM, SSD

Explanation: Đại diện cho bộ nhớ bán dẫn là ROM, RAM và SSD, do chất bán dẫn có tốc độ truy cập cao

(slide trang 8 chương 4 phần 5)

Question 38: Control Unit (CU) uses clock to synchronize activities of functional units inside CPU and between _____ components

a. CPU and GPU

b. CPU and internal

c. CPU and external

d. Internal and external

Explanation: Bộ điều khiển CU sử dụng nhịp đồng hồ (clock) để đồng bộ (synchronize) các đơn vị chức năng trong CPU và giữa CPU với các bộ phận bên ngoài (external component)

(Trang 21 giáo trình, slide trang 19 chương 2)

Question 39: What is the number of words can be transferred per clock cycle of DDR3 SDRAM?

a. 8

b. 6

c. 2

d. 4

Explanation: DDR3 SDRAM có khả năng chuyển 8 từ dữ liệu (word) trong 1 chu kỳ đồng hồ (clock cycle)

(Trang 45 giáo trình chương 4, slide trang 20 chương 4 phần 5)

Question 40: Which is the increment order of memory components in sizes?

a. ROM, RAM, registers, cache, HDD

b. ROM, RAM, HDD, registers, cache

c. Registers, ROM, RAM, cache, HDD

d. Registers, Cache, ROM, RAM, HDD

Explanation: Nếu sắp xếp theo dung lượng, các bộ nhớ sẽ được xếp như sau: thanh ghi, cache,bộ nhớ chính (ROM, RAM), bộ nhớ ngoài

(Trang 60-73 giáo trình chương 5)

Question 41: What types of pipeline conflicts is it which where are data dependency among consecutive instructions?

a. Branching conflict

b. Data conflict

c. Resource conflict

d. No conflict

Explanation: Ví dụ trong giáo trình giải thích rõ

(Trang 35 giáo trình chương 3, slide trang 14 chương 3 phần 4)

Question 42: The computer input and output devices are also called _____ devices

a. external

b. internal

c. peripheral

d. periphered

Explanation: Thiết bị ngoại vi (peripheral devices) gồm các thiết bị vào và các thiết bị ra

(Trang 8 giáo trình, slide trang 16 chương 1)

Question 43: Why can multi-level cache give better performance than one level cache?

a. Because it can balance the speed of CPU and main memory better

b. Because it can make the speed of main memory faster

c. Because it can balance the speed of CPU and RAM memory better

d. Because it can balance the volume of CPU and main memory better

(Trang 226)

Question 44: What are the characteristics of magnetic disks?

a. Volatile, hard storage, based on magnetic principles

b. Non-volatile, mass storage, based on ferromagnetic principles

c. Non-volatile, mass storage, based on magnetic principles

d. Volatile, hard storage, based on ferromagnetic principles

Explanation: Đĩa từ (magnetic disk) có các tính chất sau: (1) thuộc loại bộ nhớ ổn định (non-volatile), tức thông tin trên đĩa không phụ thuộc vào nguồn điện nuôi bên ngoài, (2) là bộ nhớ kiểu khối (mass storage) và (3) hoạt động dựa trên nguyên lý từ (magnetic principle)

(Trang 60 giáo trình chương 5, slide trang 3 chương 4 phần 6)

155

Question 45: What are functional keys on standard keyboards?

a. F1, F3, F5, F9, Fn

b. F1, F3, F5, F9, F11

c. F2, F4, F6, Fn, F8

d. F1, F2, F3, F4, Fn

Explanation: Bàn phím tiêu chuẩn có 101 phím: ký tự a-z, số 0-9, cóc phím toán, các phím chức năng F1-F12_____ Fn không phải là phím chức năng (functional key)

(Trang 84 giáo trình chương 6, slide trang 11 chương 5 phần 8)

Question 46: The roles of cache memory are to improve the _____ and to reduce the _____

a. performance / costs

b. system performance / manufacturing expensive

c. system performance / manufacturing costs

d. CPU performance / manufacturing costs

Explanation: Cấu trúc phân cấp trong hệ thống nhớ có hai vai trò chính: (1) tăng hiệu năng hệ thống (system performance) và (2) giảm giá thành sản xuất (manufacturing cost)

(Trang 60-73 giáo trình chương 5)

Question 47: Why is cache considered a smart memory?

a. Because cache can load enough instructions and data for CPU

b. Because cache can predict the need of instructions and data of CPU

c. Because cache can reduce the memory access time

d. Because cache is a very fast memory

Explanation: Cache được coi là bộ nhớ “thông minh” vì: (1) nó có thể dự đoán được nhu cầu lệnh và dữ liệu của CPU và (2) nó tải trước các lệnh và dữ liệu CPU cần sử dụng từ bộ nhớ chính, giúp giảm thời gian truy cập hệ thống nhớ, tăng tốc độ xử lý

(Trang 41 giáo trình chương 4, slide trang 26 chương 4 phần 5)

Question 48: What are the advantages of LCD over CRT monitors?

a. Thinner and lighter, consume less power, larger size

b. Lighter, consume less power, larger viewable size

c. Thinner and lighter, consume less power, larger viewable size

d. Thinner, consume less power, larger viewable size

Explanation: So với CRT, màn hình LCD mỏng hơn, nhẹ hơn (thinner and lighter), tiêu thụ ít điện năng hơn (consume less power), và phần diện tích màn thực để hiển thị ảnh (viewable size) cũng lớn hơn

(Trang 88 giáo trình chương 6, slide trang 30 chương 5 phần 8)

Question 49: What are the advantages of SSDs over HDDs in computers?

a. Higher accessing, less expensive, more compact in size

b. Higher accessing speed, less power consumption, more compact in size

c. Higher accessing speed, less power consumption, less compact in size

d. Bigger accessing speed, less wire consumption, less compact in size

Explanation: SSD hơn HDD ở chỗ: (1) tốc độ truy cập nhanh hơn, (2) tiêu thụ ít điện năng hơn và (3) nhỏ gọn hơn (more compact in size)

(slide trang 26 chương 4 phần 6)

Question 50: An electron gun in CRT monitors is the element that produce _____

a. light beam

b. an electron beam

c. an electronic beam

d. electrons

Explanation: Súng điện tử (electron gun) trong màn hình CRT bắt tia điện tử (electron beam) lên mặt huỳnh quang phốt pho để tạo ảnh

(Trang 87 giáo trình chương 6, slide trang 29 chương 5 phần 8)

Question 51: Generally, an instruction has two parts of opcode and _____

a. sources of operands

b. destinations of operands

c. values of operands

d. addresses of operands

Explanation: Dạng tổng quát của lệnh máy tính gồm 2 thành phần chính: mã lệnh (opcode) và địa chỉ của các toán hạng (addresses of operands)

(Trang 9 giáo trình chương 3, slide trang 7 chương 3 phần 3-p95)

Question 52: After CPU executes one instruction, the _____ is loaded into PC register

a. memory address of the next instruction

b. next instruction of memory address

c. memory address

d. next instruction

Explanation: Sau khi CPU thực thi xong một lệnh, địa chỉ của lệnh tiếp theo được nạp vào thanh ghi PC

(Trang 19 giáo trình chương 3, slide trang 5 chương 3 phần 3)

Question 53: What is the function of computer mice?

a. Input data and control

b. Control

c. Input data

d. Input data or control

Explanation: Chức năng chính của chuột là điều khiển

(Trang 85 giáo trình chương 6, slide trang 17 chương 5 phần 8)

Question 54: What is the meaning of the instruction “ADD R3, R2, R1” where Ri is a CPU register?

a. R1 ⟵ R2 + R3

b. R2 ⟵ R3 + R1

c. R3 ⟵ R3 + R2 + R1

d. R3 ⟵ R2 + R1

Explanation: Công thức lệnh cộng: “ADD R3, R2, R1” tức R3 ⟵ R2 + R1, tức cộng nội dung 2 thanh ghi R2 và R1 và lưu kết quả vào thanh ghi R3

(Trang 31 giáo trình chương 3p96)

Question 55: Addressing modes are methods that _____ are organized

a. instruction elements

b. command elements

c. instruction operands

d. instruction parts

Explanation: Chế độ địa chỉ là cách thức CPU tổ chức các toán hạng của lệnh (instruction operand)

(Trang 26 giáo trình chương 3, slide trang 13 chương 3 phần 3 - p101)

Question 56: Computer organization is the science that studies _____

a. computer structures and their working methodologies

b. computer components and their running methodologies

c. computer structures and their using methodologies

d. computer components and their working methodologies

Explanation: Tổ chức máy tính (computer organization) là khoa học nghiên cứu về các bộ phận máy tính (computer component) và cách thức làm việc của chúng (working methodology)

(Trang 8 giáo trình, slide trang 6 chương 1)

Question 57: Which is a correct name of SDRAM?

a. Synchronous DRAM

b. Statistical DRAM

c Static DRAM

d. Synchrone DRAM

Explanation: SDRAM – Synchronous Dynamic RAM, RAM động đồng bộ

(Trang 44 giáo trình chương 4, slide trang 183 chương 4 phần 5)

Question 58: How many electron beams that the color monitor uses to generate a pixel of a picture?

a. 1

b. 2

c. 3

d. 4

Explanation: Đối với màn hình CRT (CRT monitor), màn hình đen trắng thì dùng 1 súng điện tử (electron beam), còn màn hình màu dùng 3 súng điện tử (RGB) để tạo ra các điểm ảnh

(Trang 87 giáo trình chương 6, slide trang 354 chương 5 phần 8)

Question 59: What are the types of ROM memory?

a. PROM, EPROM, EEPROM, Flash memory

b. PROM, EPROM, ECPROM, Flash memory

c. ROM, EPROM, EEPROM, Flash memory

d. PROM, EPROM, EEPROM, USB

Explanation: ROM gồm các loại sau: ROM nguyên thủy (Ordinary ROM), ROM có thể lập trình (Programmable ROM – PROM), PROM có thể xóa được (Erasable PROM – EPROM), PROM có thể xóa được bằng điện (Electrically Erasable PROM - EEPROM), bộ nhớ Flash (Flash memory)

(Trang 42 giáo trình chương 4, slide trang 176 chương 4 phần 5)

Question 60: Which is NOT a correct statement about instruction execution cycle and its components of traditional CPUs?

a. An execution stage may consist of some execution phases.

b. An execution stage may consist of some machine cycles.

c. One machine cycle consists of some clock cycles.

d. An instruction execution cycle consists of some instruction execution stages.

Explanation: Chu trình xử lý lệnh (instruction execution cycle) là một chu trình mà CPU hoàn thành xử lý lệnh và được chia nhỏ ra như sau: (1) chu trình được chia thành các giai đoạn xử lý lệnh (instruction execution stage), (2) mỗi giai đoạn xử lý lệnh gồm vài chu trình máy (machine cycle), và mỗi chu trình máy gồm vài chu trình đồng hồ (clock cycle)

(Slide trang 93 chương 3 phần 3)

Question 61: What are two basic techniques used in RAID storage system?

a. Disk striping, Disk mirroring

b. Disk strip, Disk mirror

c. Disk straping, Disk mirroring

d. Disk striping, Disk rotating

(Slide trang 281)

Question 62: What are types of optical disks?

a. CD-ROM, CD-R, CD-RW, DVD-ROM, DVD-R, DVD-RW

b. CD-RAM, CD-R, CD-RW, DVD-RAM, DVD-R, DVD-RW

c. CD, CD-R, CD-RW, DVD, DVD-R, DVD-RW

d. CD-ROM, CD-R, CD-W, DVD-ROM, DVD-R, DVD-W

(Slide trang 269)

Question 63: In CPU pipeline, the delayed branching technique is suitable for _____

a. medium pipelines.

b. long pipelines.

c. short pipelines.

d. very long pipelines.

(slide trang 147)

Question 64: Which is NOT a drawback of cache “Look Through” architecture?

a. Hit is slow

b. Complicated design

c. Miss is slow

d. Expensive

(Slide trang 195)

﻿

Question 65: Computer memory components are usually classified based on _____

a. access types, information capacity, manufacturing technologies.

b. access types, information sustainability, manufacturing technologies.

c. access speeds, information sustainability, manufacturing technologies.

d. access types, information sustainability, capacity.

(Slide trang 171)

Question 66: What are the characteristics of the cache set associative mapping technique?

a. Simple, slow and flexible mapping

b. Complicated, slow and flexible mapping

c. Simple, fast and fixed mapping

d. Complicated, fast and flexible mapping

(Slide trang 212)

Question 67: What are common SSD components?

a. Flesh memory, controller, Cache RAM, M.2 interface

b. Flesh memory, Cache controller, Cache RAM, Connection interface

c. Flash memory, Controller, Cache, Connection interface

d. Flash memory, Controller, Cache, SATA interface

(Slide trang 260)

Question 68: CRT monitor use the _____ emitted from its cathode to _____ layers to create pictures.

a. eletric beam / phosphor

b. electronic beam / phosphor

c. electron beam / phosphor

d. eletron beam / phophor

(Slide trang 354)

Question 69: What is NOT a component of the instruction set architecture?

a. Address and data format

b. Processor registers

c. The instruction set

d. Memory Addressing (thiếu “modes”)

(Slide trang 7)

Question 70: What are logical elements of HDDs of computer systems?

a. Partitions, sectors, files, directories

b. Partition table, sectors, files, root directory

c. Partition table, boot sector, file system, root directory

d. Partitions, root sector, file system, boot directory

(Slide trang 252-254)

Question 71: What are main components of a laser printer?

a. ﻿Charger for drum, Charger for paper, Drying drum, Paper tray

b. Electrode charger for drum, Electrode charger for paper, Heating drum, Paper track

c. Electrode charger for drum, Electrode charger for paper, Drying drum, Paper tray

d. Charger for drum, Charger for paper, Drying drum, Paper track

Explanation: Tham khảo câu 8

Question 72: What are correct typical addressing modes?

a. Immediote, Direct, Index, Relative

b. Immediote. Direct, Index, Relate

c. Immediate, Direct, Indexed, Memory

d. Immediate, Direct, Indexed, Relative

Explanation: Có 6 chế độ địa chỉ thông dụng: tức thì (immediate), trực tiếp (direct), gián tiếp qua thanh ghi (register indirect), gián tiếp qua ô nhớ (memory indirect), chỉ số (indexed), tương đối (relative)

(Trang 27 giáo trình chương 3, slide trang 101 chương 3 phần 3)

Question 73: What is the correct name of AGP bus?

a. Advanced Graph Port

b. Advanced Graphics Port

c. Accelerated Graphics Port

d. Accelerated Graph Port

Explanation: AGP – accelerated graphics port, cổng tăng tốc đồ họa

(Trang 7 giáo trình, trang 80 giáo trình chương 6, slide trang 318 chương 5 phần 7)

Question 74: What is the elements that SSDs use to store data?

a. solid-state flash memory chips

b. RAM memory chips

c. solid-state flesh memory chips

d. ROM memory chips

Explanation: SSD sử dụng chip bộ nhớ flash thể rắn (solid-state flash memory chip) để lưu trữ dữ liệu

(Slide trang 255 chương 4 phần 6)

Question 75: What is the cache replacement policy that has the lowest miss rate?

a. Random

b. LIFO

c. FIFO

d. LRU

(slide trang 219)

Question 76: What are the characteristics of the cache direct mapping technique?

a. Simple, fast and fixed mapping

b. Simple, slow and flexible mapping

c. Complicated, slow and flexible mapping

d. Complicated, fast and flexible mapping

(slide trang 197)

Question 77: What are the common sizes of CPU registers?

a. 32, 64, 128, 256

b. 4, 8, 16, 32

c. 8, 16, 32, 64

d. 16, 32, 64, 128

(slide 62)

Question 78: ﻿ What are the capacity of standard CD-ROM and DVD-ROM?

a. 750MB and 4.2GB

b. 650MB and 4.5GB

c. 720MB and 4.8GB

d. 700MB and 4.7GB

(slide 270- 274)

Question 79: What is the number of words can be transferred per clock cycle of DDR2 SDRAM?

a. 2

b. 4

c. 8

d. 6

(slide trang 183)

Question 80: What are bit widths of ISA-AT and EISA buses?

a. 8 and 16

b. 24 and 32

c. 16 and 32

d. 16 and 24

﻿(slide trang 304-306)

Question 81: In CPU pipeline, the instruction execution is divided into stages and _____ is executed by _____

a. a CPU component / each stage.

b. CPU components / stages.

c. stages / CPU components.

d. each stage / a CPU component.

(Slide trang 125)

Question 82: Common data movement instructions include _____

a. MOV, LOD, STORE, PUSH.

b. MOVE, LOAD, STORE, PUSH.

c. MOVE, LOAD, STO, POP.

d. MOVE, LOAD, STORE, PUT.

(Trang 112)

Question 83: Size of CPU Accumulator (A) register is equivalent to CPU_____

a. data size.

b. word length.

c. data word.

d. data word length.

(Slide trang 63)

Question 84: ﻿Cache exchanges _____ with CPU while it exchanges _____ with main memory.

a. individual data items / data blocks

b. data blocks / individual data items

c. individual data / blocked data

d. instruction items / instruction blocks

(Slide trang 192)

Question 85: What are common arithmetic instructions supported by ALU?

a. ADD, SUB, DIV, MUL, SHR, ROL

b. ADD, SBB, DIV, MUR, SHR, ROL

c. ADD, SUB, DIV, MUL, SHF, ROF

d. ADB, SUB, DIV, MUL, SHR, ROR

(Slide trang 76)

Question 86: What is the addressing mode of the instruction "LOAD R1, (1000)", where Ri is a CPU register?

a. Indirect

b. Direct

c. Relative

d. Immediate

(Trang 109)

Question 87: ﻿A solution for memory access conflicts in CPU pipeline is to _____ to improve concurrent accesses.

a. upgrade memory capacity of the system

b. split cache into multiple levels

c. add more RAM into the system

d. split L1 cache into I-Cache and D-Cache

(Slide trang 133)

Question 88: What are common interconnect interfaces of SSDs to computer systems?

a. ATA, SATA, M.2, U.2, PCI

b. SAS, SATA, M.2, U.8, PCIX

c. SAS, PATA, M., UB, PCle

d. SAS, SATA, M.2, U.2, PCIe

(Slide trang 259)

Question 89: What is the communication speed of USB 3.0 ports?

a. 1.5Gb/s

b. 2.0Gb/s

c. 3Gb/s

d. 1Gb/s

(Slide trang 332)

Question 90: If memory only supports _____ read/write operation at a time, _____ memory accesses from CPU pipeline will cause resource conflicts.

a. two/simultaneous

b. one/simultaneous

c. one / multiple

d. two/multiple

(Slide trang 132)

Question 91: ﻿In cache spatial locality, if a memory location is accessed, the probability that its _____ are accessed in _____ is high

a. near neighbours / closed future

b. closed neighbours / far future

c. closed neighbours / near future

d. neighbours / future

(Slide trang 190)

Question 92: What are cache miss reduction methods?

a. Increase cache block size, Increase cache associative level

b. Increase cache block size, Increase cache level

c. Increase cache size, Increase cache associative level

d. Increase cache size, Increase cache level

(Slide trang 229)

Question 93: Which is a true statement about the cost role of cache memory?

a. If two systems have the same performance, the system with cache is cheaper.

b. If two systems have the same cost, the system without cache is faster.

c. If two systems have the same cost, the system with cache is simpler.

d. If two systems have the same performance, the system with cache is more expensive.

(Slide trang 188)

Question 94: What are clock speeds of ISA buses?

a. 8MHz, 10MHz, 12MHz, 14MHz

b. 6MHz, 8MHz, 10MHz

c. 4MHz, 6MHz, 8MHz

d. 2MHz, 4MHz, 6MHz

(Slide trang 304)

Question 95: What are correct types of RAID storage systems?

a. RAID 1, RAID 3, RAID 5, RAID 7

b. RAID 3, RAID 5, RAID 7, RAID 9

c. RAID 0, RAID 2, RAID 4, RAID 6

d. RAID 2, RAID 4, RAID 6, RAID 8

(Slide trang 284)

Question 96: What are 4 general registers of the Intel 8086 CPU?

a. AX, BX, CX, DX

b. A, X, Y, Z

c. AX, BX, SI, DI

d. A, B, C, D

(Slide trang 70)

Question 97: ﻿Liquid crystals in LCD are semi-solid substances that are sensitive to _____

a. temperature or electricity.

b. temperature and electronics.

c. temperature and electricity.

d. heat and electricity.

(Slide trang 355)

Question 98: What are the functions of computer keyboards?

a. Output data and Control

b. Input data and Control

c. Input data and Storage

d. Import data and Storage

Question 99: What is the meaning of the instruction “SUBTRACT R1, R2, R3” where Ri is a CPU register?

a. R1 ⟵ R2 – R3

b. R3 ⟵ R1 – R2

c. R1 ⟵ R1 – R2 – R3

d. R2 ⟵ R1 – R3

Question 100: In computer systems, _____ numbering system is used to represent data

a. decimal

b. octal

c. binary

d. Hexadecimal

(Slide trang 43)

Question 101: What is the RAID level in the following image?

a. RAID 2

b. RAID 3

c. RAID 1

d. RAID 0

(Slide trang 285)

Question 102: Each bit of FR register stores the _____ of the operation executed by ALU

a. state of the result

b. status

c. result

d. status of the result

(Slide trang 66)

Question 103: ﻿What are common interconnect interfaces of HDDs to computer systems?

a. PATA, KATA, SCSI, MAS

b. PATA, SATA, SCSI, SAS

c. PATA, SATA, SCSI, SAS, USB

d. ATA, SATA, SCSI, SAS, USB

(Slide trang 242)

Question 104: An electron gun in CRT monitors is the element that produces _____

a. electrons.

b. an electronic beam.

c. an electron beam.

d. light beam.

(Slide trang 351)

Question 105: Which is NOT a cache organization technique?

a. Fully associative mapping

b. Indirect mapping

c. Set associative mapping

d. Direct mapping

(Slide trang 197)

Question 106: ﻿Which is NOT a correct statement about CPU instructions?

a. Instructions are stored in memory.

b. One instruction has its own function.

c. Instructions are fetched from memory to CPU for execution.

d. Instructions are saved from CPU to memory after execution.

(Slide trang 91)

Question 107: Which is NOT a true memory type based on manufacturing technologies?

a. Semiconductor memory

b. Volatile memory

c. Optical memory

d. Magnetic memory

(Slide trang 171)

Question 108: Computer bus is _____ that transfer data between _____ inside a computer

a. a subsystem / CPU and memory

b. a system / components

c. a system / CPU and memory

d. a subsystem / components

(Slide trang 300)

Question 109: ﻿What are the elements of cache direct mapping address?

a. Tag, Line, Word

b. Tag, Line, Byte

c. Tab, Line, Word

d. Tag, Lane, Word

(Slide trang 200)

Question 110: Laser printers work on the principle of _____ by laser beam.

a. electronic imaging

b. electronic photo

c. electric imaging

d. electric photo

(Slide trang 368)

Question 111: What are the solutions for conditional branching conflicts of the CPU pipeline?

a. Delayed branching, Branch cancellation

b. Delayed branching, Branch prediction

c. Cancelled branching, Branch prediction

d. Cancelled branching, Branch cancellation

(Slide trang 141)

Question 112: ﻿What is the correct name of TFT in LCD monitors?

a. Thick Film Transistors

b. Thick Field Transistors

c. Thin Film Transistors

d. Thin Field Transistors

(Slide trang 357)

Question 113: What is NOT the correct statement about ROM and RAM?

a. ROM is read only memory and RAM is random access memory.

b. ROM capacity is usually smaller than RAM capacity.

c. ROM is only used to store instructions while RAM is only used to store instructions and data.

d. The information can only be read from ROM while the information can be read from and write to RAM.

(Slide trang 14)

Question 114: A SRAM flip-flop usually uses _____ transistors.

a. 4, 6, 8

b. 8, 10, 12

c. 4, 8, 12

d. 6, 8, 10

(Slide trang 180)

Question 115: ﻿Three components of computer architecture are_____

a. Instruction set architecture, Microarchitecture, System Design.

b. Instruction set architecture, Macroarchitecture, System Design.

c. Instruction set architecture, Microarchitecture, Analysis and Design.

d. Instruction architecture, Microarchitecture, System Design.

(Slide trang 7, 8, 9)

Question 116: What are types of computer mice?

a. Mechanical mouse, Red light mouse, Laser mouse

b. Electronic mouse, Optical mouse, Violet mouse

c. Mechanical mouse, Optical mouse, Laser mouse

d. Electric mouse, Red light mouse, Laser mouse

(Slide trang 342)

Question 117: Network attached storage is _____ which is dedicated for _____

a. a machine / storage.

b. a server / storing information.

c a server / storage.

d. a machine / storing information.

(Slide trang 292)

Question 118: Storage Area Network (SAN) is _____ which is dedicated for storage services.

a. a network of servers

b. a number of machines

c. a number of servers

d. a group of servers

(Slide trang 295)

Question 119: What are the status flags of the FR register?

a. CF, OF, AF, ZF

b. TF, OF, AF, ZF

c. CF, OF, AF, IF

d. CF, DF, AF, ZF

(Slide trang 66)

Question 120: Which is the cache level that is usually divided into 2 parts of I-cache and D-cache?

a. Level 0

b. Level 1

c. Level 2

d. Level 3

(Slide trang 133)

Question 121: What are 2 LCD types based on the light sources?

a. Backlight and Reflect

b. Backlight and Reflective

c. Backlit and Reflective

d. Backlit and Reflect

Question 122: What is the meaning of the instructions "LOAD 1000, (R1)", where R1 is a CPU register?

a. 1000 → R1

b. M[1000] → M[R1]

c. M[1000] ⟵ R1

d. M[1000] ⟵ M[R1] (Hơi giống chế độ địa chỉ Direct kết hợp Register Indirect, với 1000 chỉ địa chỉ ô nhớ - dạng trực tiếp, còn (R1) là ô nhớ có địa chỉ được lưu ở thanh ghi R1)

Question 123: The cache spatial locality usually applies to _____ which have highly serial order in program space.

a. instructions

b. data items and instructions

c. data items or instructions

d. data items

(Slide trang 190)

Question 124: What are types of cache misses?

a. Compalsary misses, Capacity misses, Conflict misses

b. Compulsory misses, Capacity misses, Size misses

c. Compulsory misses, Capacity misses, Conflict misses

d. Compulsory misses, Storage misses, Conflict misses

(Slide trang 227)

Question 125: Cache works based on 2 basic principles, including _____

a. space locality and time locality

b. space locality and temporal locality

c. spatial locality and temporal locality

d. spatial locality and time locality

Question 126: What is the addressing mode of the instruction “LOAD Ri , X(PC)” where Ri is a CPU register?

a. Relative

b. Indirect

c. Direct

d. Indexed

(Trang 109)

Question 127: What are the characteristics of RAM memory?

a. Non-volatile memory, semiconductor memory, random access memory

b. Volatile memory, magnetic memory, read only memory

c. Volatile memory, semiconductor memory, random access memory

d. Non-volatile memory, semiconductor memory, random access memory

(Trang 177)

Question 128: What is the computer component that fetches instructions from memory, decodes and executes instructions?

a. CU

b. ALU

c. Registers

d. CPU

(Trang 33)

Question 129: Which is NOT a correct cache replacement policies?

a. Random

b. LRU

c. FIFO

d. LIFO

(Trang 216)

Question 130: Which is the increment order of memory components in accessing speed?

a. RAM, cache. registers, HDD, SSD

b. SSD, HDD, RAM, cache, registers

c. HDD, SSD, RAM, cache, registers

d. HDD, SSD, RAM, registers, cache

Question 131: ﻿0 address of operands is used in instructions that perform _____ operations, such as push and pop.

a. string

b. queue

c. stack

d. Tree

(Trang 100)

Question 132: The future trend of cache is it getting bigger and bigger to better support for multi-tasking, _____

a. multi-cache and multi-core systems.

b. multi-core and multi-processor systems.

c. parallel processing and grid computing.

d. parallel processing and multi-core CPU systems.

(Trang 233)

Question 133: What are common logic instructions supported by ALU?

a. NOT, AND, NOR, XOR

b. NOT, AND, OR, XORR

c. NOT, AND, OR, XOR

d. NOT, NAND, OR, XOR

(Trang 76)

Question 133: What is the meaning of the instruction “LOAD Ri , X(PC)” where Ri is a CPU register?

a. Ri → M[X+PC]

b. Ri ⟵ M[X+PC]

c. Ri ⟵ X + M[PC]

d. Ri ⟵ X+PC

(Trang 109)

Question 134: ﻿What are the components of CPU in the general block diagram?

a. CPU, ALU, PC, IR, FR

b. CU, ALU, PC, IR, FR

c. CU, GPU, PC, IR, FR

d. CU, ALU, PR, IR, FR

(Trang 59)

Question 135: What are major elements of cache memory?

a. SRAM, Cache controller, Tag RAM

b. SDRAM, Cache controller, Tag RAM

c. DRAM, Controller, Tag RAM

d. SRAM, Cache controller, RAM

(Trang 195)

Question 136: How many cache levels are there in most modern CPUs?

a. 3

b. 4

c. 2 (Không chắc)

d. 1

Question 137: What is an output peripheral device for computer?

a. Mouse

b. Scanner

c. Keyboard

d. CD-R drive

Question 138: What is the number of addresses of operands of the instruction “ADD R1, A” where R1 is a CPU register and A is a memory location?

a. 1.5

b. 2

c. 1

d. 2.5

Explanation: Câu lệnh gồm thanh ghi và địa chỉ ô nhớ thuộc dạng 1.5 địa chỉ

(Trang 99)

Question 139: Control Unit (CU) gets instruction from _____ register, decodes and generates signal to control other components of the system.

a. PC

b. MAR

c. IP

d. MBR

(Trang 75)

Question 140: ﻿What are the factors that affect cache performance?

a. Cache size, cache split, multi-level cache

b. Cache size, cache separation, cache levels

c. Cache size, cache aggregate, multi-level cache

d. Cache size, cache split, cache levels

(Trang 221)

Question 141: Which is a true statement about the position of cache in memory system?

a. Cache usually resides inside the CPU.

b. Cache must reside inside the CPU.

c. Cache can be located anywhere in a computer system.

d. Cache usually resides outside the CPU.

Question 142: ﻿Common groups of CPU instructions include _____

a. data movement group, arithmetic group, conditional and branching group.

b. data movement group, computational group, conditional and looping group.

c. data movement group, computational group, conditional and branching group.

d. data processing group, computational group, conditional and branching group.

(Trang 91)

Question 143: What are the correct 3 major components of a basic CPU?

a. CU, ALU, Cache

b. CU, Cache, Registers

c. CU, ALU, Registers

d. GPU, ALU, Registers

(Trang 58)

Question 144: Cache is a memory element in the computer memory hierarchy and it acts as the _____ between CPU and the main memory.

a. intermediary

b. immediate

c. storage

d. Memory

(Trang 186)

Question 145: What are NOT elements of an instruction execution cycle?

a. Memory read cycle, Memory write cycle, Peripheral read cycle, Interrupt acceptance cycle

b. Memory read cycle, Memory write cycle, Peripheral read cycle, Bus free cycle

c. Instruction fetch cycle, Memory read cycle, Peripheral read cycle, Bus read cycle

d. Instruction fetch cycle, Memory read cycle, Peripheral read cycle, Bus free cycle

(Trang 94)

Question 146: What are the elements of cache set associative mapping address?

a. Tag, Set, Byte

b. Tag, Set, Word

c. Tag, Line, Word

d. Tab, Line, Word

(Trang 210)

Question 147: What is the technique used in RAID 1?

a. Disk striping or disk mirroring

b. Disk striping

c. Disk mirroring

d. Disk striping and disk mirroring

(Trang 288)

Question 148: Why is DRAM usually cheaper than SRAM?

a. Because DRAM uses less capacitors per memory cell than SRAM.

b. Because DRAM uses more capacitors per memory cell than SRAM.

c. Because DRAM uses less transistors per memory cell than SRAM.

d. Because DRAM is simpler than SRAM.

(Trang 180)

Question 149: What are the inputs to the control unit (CU)?

a. CLK, IR, flags

b. CLK, IR, ALU flags

c. CLK, PC, ALU flags

d. CLK, IR, ALU

(Trang 74)

Question 150: A nibble is a group of _____ bits

a. 2

b. 8

c. 4

d. 6

(Trang 48)

Question 151: Common control and sequencing instructions include _____

a. JUMP, CALL, RET, BRANCH-CONDITION

b. JUMP, CALL, RETURN, BRANCH-IF-CONDITION

c. JUMP, CALL, RETURN, BRANCH-IF

d. JMP, CALL, RET, BRANCH-IF-CONDITION

(Trang 117)

Question 152: ﻿Instruction execution cycle is the _____ that CPU completes the execution of the instruction.

a. clock of time

b. periodic time

c. period of time

d. number of clocks

(Trang 93)

Question 153: SRAM is faster than DRAM because its bits have the _____ and SRAM chips accept all address bits at a time.

a. symmetric structure

b. complex architecture

c. simple structure

d. symmetric architecture

(Trang 180)

Question 154: What is the addressing mode of the instruction “LOAD Ri , X(Rind)” where Ri is a CPU register?

a. Relative

b. Indexed

c. Indirect

d. Direct

(Trang 109)

Question 155: When a key is pressed, the keyboard controller detects the event and generates a corresponding _____ code

a. UTF-8

b. scan

c. char

d. ASCII

(Trang 341)

Question 156: What is the addressing mode of the instruction “LOAD R1 , (R2)” where Ri is a CPU register?

a. Relative

b. Indexed

c. Indirect

d. Direct

(Trang 109)

Question 157: What is the correct full name of SSD in computers?

a. Solid-State Drive

b. Solid-State Driver

c. Soulid-State Driver

d. Solid-Status Drive

(Trang 255)

Question 158: What are the characteristics of CPU registers?

a. Fast, small and size

b. Very fast, small in size and low frequency

c. Very fast, small in counter and size

d. Very fast, small in number and size

(Trang 62)

Question 159: What are the purposes in designing RAID storage system?

a. High speed, high durability, large size

b. High performance, high reliability, large volume

c. High performance, high reliability, large size

d. High speed, high durability, large volume

(Trang 280)

Question 160: What are the characteristics of cache memory?

a. fast, small in size, low cost per bit

b. very fast, small in size, very high cost per bit

c. very fast, large in size, high cost per bit

d. very fast, small in size, high cost per bit

(Trang 168 + 187)

Question 161: SP or Stack Pointer is the register that always points to _____ of the stack

a. the middle

b. the bottom

c. the top

d. next frame

(Trang 69)

Question 162: Microarchitecture is _____ level description of the system

a. a lower

b. multiple

c. a higher

d. a middle

(Trang 8)

Question 163: Input and Output (I/O) instructions are used to transfer data between _____ and _____

a. the computer / internal devices.

b. the CPU / external devices.

c. the CPU / peripheral devices.

d. the computer / peripheral devices.

(Trang 119)

Question 164: What are the elements of a HDD in computer systems?

a. Disks, heads, racks, sectors, cylinders

b. Disks, head, tracks, sectors, cilinders

c. Disk, head, rack, sector, cylinder

d. Disks, heads, tracks, sectors, cylinders

(Trang 238 - 241)

Question 165: Two types of cache architecture are _____

a. Look Inside and Look Thorough

b. Look Inside and Look Through

c. Look Aside and Look Through

d. Look Aside and Look Thorough

(Trang 195 - 196)

Question 166: Given two instructions: “ADD R1, R1, R3” and “SUB R4, R1, R2”, where Ri is a CPU register. What is the CPU pipeline conflict with these instructions?

a. Resource conflict

b. Data conflict

c. No conflict

d. Branching conflict

(Trang 134)

Question 167: What are types of computer monitors?

a. CRT, LCD, OLED

b. CRT, LCD, Retina

c. CRT, LCD, Plasma

d. CRT, LCD, Amoled

(Trang 349)

Question 168: What are common functions of a multi-function printer?

a. Print, scan, copy

b. Print, scan, phone

c. Print, scan, messaging

d. Print, phone, copy

(Cơ bản là làm gì có máy in nào dùng để gọi và nhắn tin)

Question 169: Why does DRAM memory need periodic refresh?

a. Because information stored in DRAM is removed automatically.

b. Because electric charge stored in DRAM is removed automatically.

c. Because electric charge in the transistor of DRAM bits is leaking by nature.

d. Because electric charge in the capacitor of DRAM bits is leaking by nature.

(Trang 182)

Question 170: System Design which includes all of the other hardware components within a computing system, such as_____

a. System interconnects and CPU off-load mechanisms.

b. System connects and CPU off-load mechanisms.

c. System interconnects and CPU on-load mechanisms.

d. System connects and CPU on-load mechanisms.

(Trang 9)

Question 171: What are 2 LCD types based on the control methods?

a. Passed matrix and Active matrix

b. Passive matrix and Active matrix

c. Passive matrix and Acted matrix

d. Passing matrix and Acting matrix

(Trang 357)

Question 172: What are the CPU registers?

a. PC, IR, MBR, A B. C

b. PC, IR, MBR, A, Y, Z

c. PK, IR, MRR, A, Y, Z

d. PC, RC, MCR, A, Y, Z

(Trang 59)

Question 173: What are phases of a PCI transaction?

a. Arbitration, Address, Data

b. Arbitration, Access, Data

c. Arbitration, Address, Transfer

d. Arbiter, Address, Data

(Trang 313)

Question 174: Which is the name of the program stored in HDD boot sector?

a. Bootstraper

b. Rootstraper

c. Bootstrap loader

d. Rootstrap loader

(Trang 253)

Question 175: Each bit of SRAM is based on _____ while each bit of DRAM is based on _____

a. a flip-flop / a capacitor

b. a transistor / a capacitor

c. flip-flops / capacitors

d. transistors / a capacitors

(Trang 178)

Question 176: What are types of printers?

a. Typed-writer, Pin-matrix, Laser, Inkjet

b. Typewriter-derived, Dot-matrix, Laser, Inkjet

c. Typed-writer, Dot-matrix, Laser, Inkjet

d. Typewriter-derived, Pin-matrix, Laser, Inkjet

(Trang 361)

Question 177: Hit is an event, in which CPU accesses a data item that is _____ in cache and miss is an event, in which CPU accesses a data item that is _____ in cache

a. found / not found

b. not found / found

c. ready / not ready

d. available / not available

(Trang 193)

________________________ (16:25 26/5/23) ________________________

Question 178: System buses are sets of wires to connect CPU with other components of a computer. System buses consist of_____

a. A bus, B bus and C bus.

b. A bus, C bus and D bus.

c. B bus, D bus and C bus.

d. B bus, C bus and D bus.

Explanation: Hệ thống bus gồm 3 loại bus: bus địa chỉ (Address Bus - Bus A), bus điều khiển (Control bus - Bus C), bus dữ liệu (Data Bus - Bus D)

(Trang 18)

Question 179: IR or Instruction register is used to store the _____ instruction

a. current executed

b. executed

c. next executed

d. next

Explanation: Thanh ghi lệnh IR lưu lệnh đang được thực hiện (current executed instruction)

(Trang 71)

Question 180: What are the components of the PCI block diagram?

a. ABH master, ABH slave, PCI referee, PCI interconnect

b. ABH server, ABH client, PCI referee, PCI interconnect

c. ABH master, ABH slave, PCI arbiter, PCI interface

d. ABH server, ABH client, PCI arbiter, PCI interface

(Trang 311)

Question 181: The cache temporal locality usually applies to data items and instructions in a _____ of program space

a. segment

b. while loop

c. loop

d. for loop

(Trang 191)

Question 182: What type of pipeline conflicts is it, in which a program have instructions, such as JUMP, CALL and RETURN?

a. resource conflict

b. data conflict

c. branching conflictprogram

d. no conflict

(Không thấy đáp án trong slide hay tài liệu, nhưng các câu lệnh đều liên quan cao đến RẼ NHÁNH - branching nên chắc là xung đột rẽ nhánh. ChatGPT cũng bảo thế)

Question 183: What are components participated in reading information from CD-ROMs?

a. Laser diot, beam split, laser source, rotation mirror

b. Laser, splitter, sensor, mirror

c. Laser diode, beam splitter, laser source, rotation mirror

d. Laser diode, beam split, laser source, mirror rotation

(Trang 268, đề cập đến laser diode, beam splitter và rotation mirror)

Question 184: Which is NOT a true memory type based on access types?

a. Serial Access Memory

b. Magnetic Memory

c. Read Only Memory

d. Random Access Memory

Explanation: Dựa vào kiểu truy cập, bộ nhớ có 3 loại là RAM, ROM và SAM. Bộ nhớ từ tính (Magnetic Memory) thuộc về loại bộ nhớ dựa trên công nghệ chế tạo

(Trang 171)

Question 185: When a program is executed, the operating system loads the program code stored in _____ memory into _____ memory

a. external / internal

b. ROM / RAM

c. internal / external

d. disk / RAM

(Trang 60, chỉ biết nạp vào bộ nhớ trong, chứ không biết nạp từ đâu)

Question 186: What are the characteristics of the cache fully associative mapping technique?

a. Complicated, fast and flexible mapping

b. Simple, fast and fixed mapping

c. Complicated, slow and flexible mapping

d. Simple, slow and fixed mapping

(Trang 197)

Question 187: The number of addresses of operands of instructions may be 3, _____

a. 2.5, 2, 1 and 0

b. 2, 1, 0.5 and 0

c. 2, 1, 1.5 and 0

d. 2.5, 2, 1.5 and 1

(Trang 95)

Question 188: The patterns of _____ represent the information stored in optical disks.

a. pits and hills

b. holes and lands

c. holes and hills

d. pits and lands

Explanation: Mặt đĩa quang được khắc rãnh và mức lõm của rãnh (pits and lands) để biểu diễn các bit thông tin

(Trang 65 giáo trình, trang 265 slide)

Question 189: What is the technique used in RAID 0?

a. Disk striping or disk mirroring

b. Disk striping

c. Disk mirroring

d. Disk striping and disk mirroring

(Trang 286)

Question 190: Computer organization is a part of computer architecture and also called _____

a. system design

b. macroarchitecture

c. instruction set architecture

d. analysis and design

(Trang 8)

Question 191: What is an output device?

a. Keyboard

b. Disk drive

c. Scanner

d. Mouse

Explanation: Bàn phím (keyboard), máy quét (scanner) và chuột (mouse) đều là thiết bị vào. Ổ đĩa (disk drive) vừa là thiết bị vào, vừa là thiết bị ra

(Trang 328)

Question 192: Internal bus is the communication channel of all CPU components. It supports _____ ways of communication

a. four

b. three

c. two

d. one

Explanation: Bus trong CPU hỗ trợ kênh giao tiếp song công (full duplex) - giao tiếp 2 chiều

(Trang 78)

Question 193: What is the RAID level in the following image?

a. RAID 0

b. RAID 3

c. RAID 2

d. RAID 1

(Trang 287)

________________________ (17:14 26/5/23) ________________________

Question 194: Which is true statement about the performance role of cache memory?

a. Cache balances the fast speed of CPU and the slow speed of main memory

b. Cache reduces the fast speed of CPU to meet the slow speed of main memory

c. Cache increases the speed of main memory to meet the CPU speed

d. The average access time of the memory system is near RAM access time

(Trang 170)

Question 195: CPU pipeline issues include _____

a. Resource conflicts, data conflicts and branching instructions

b. Resource conflicts, data conflicts and looping instructions

c. Resource conflicts, memory conflicts and branching instructions

d. Memoryconflicts, data conflicts and branching instructions

(Trang 131)

Question 196: Common arithmetic instructions include _____

a. ADC, SUBSTRACT, MULTIPLY, DIVIDE

b. ADD, SUB, MUL, DIV

c. ADD, SUBSTRACT, MULTIPLY, DIVIDE

d. ADD, SUB, MULTIPLY, DIVIDE

(Trang 76 hoặc 114, khác với câu 85 vì đấy là câu lệnh được supported bởi ALU, nhưng ở đây là câu lệnh nói chung nên chắc là C có khả năng đúng hơn)

Question 197: Computer architecture is the science and art of selecting and interconnecting hardware components to create computers that meet _____ goals

a. functions, performance and cost

b. functionality, efficiency and cost

c. functionality, performance and cost

d. functionality, performance and expensive

(Trang 6)

Question 198: (Bonus) What is the data transfer speed of the second SATA interface generation?

a. 3 Gb/s

b. 4 Gb/s

c. 5 Gb/s

d. 6 Gb/s

Question 199: (Bonus) What is the data transfer speed of the first SATA interface generation?

a. 1.5 Gb/s

b. 2 Gb/s

c. 2.5 Gb/s

d. 3 Gb/s

Question 200: What is the addressing mode of the instruction "LOAD R1, 1000", where Ri is a CPU register?

a. Indirect

b. Direct

c. Relative

d. Immediate

(Trang 109)

Question 201: (Bonus) A word is a group of _____ bits

a. 4

b. 8

c. 16

d. 32

(Trang 50)

Question 202: (Bonus) Which of the following flags does not belong to Control flags?

a. IF

b. TF

c. ZF

d. DF

Question 203: (Bonus) What are the control flags of the FR register?

a. CF, ZF, AF

b. IF, AF, OF

c. IF, DF, ZF

d. IF, DF, TF

Question 204: (Bonus) What are data transfer speeds of v1.x, v2.0, v3.0 PCIe 1 lane?

a. 4GB/s, 8GB/s, 16GB/s

b. 4GB/s, 8GB/s, 12GB/s

c. 250MB/s, 500MB/s, 1GB/s

d. 250MB/s, 500MB/s, 750MB/s

Question 205: What is the CD-ROM component that converts the reflective beam to output electric signal?

a. Laser diode

b. Rotation mirror

c. Beam splitter

d. Beam sensor

(Trang 268)

Question 206: IR gets instruction from _____ and forwards it to _____ for decoding

a. ROM / RAM

b. MAR / ALU

c. MBR / CU

d. PC / RAM

(Trang 71)

Question 207: What are types of flags?

a. Status flags, control flags, data flags

b. Status flags, control flags, condition flags

c. Status flags, control flags

d. Status flags, control flags, address flags

(Trang 66)

Question 208: What is NOT correct about internal bus?

a. The communication channel of all bus components

b. Large bandwidth

c. Support full duplex

d. Faster than external buses

Explanation: Bus trong CPU là kênh giao tiếp giữa các bộ phận trong CPU chứ không phải các bộ phận bus

(Trang 78)

Question 209: MAR gets memory address of next instruction from _____ and forwards it to _____

a. IR / control bus

b. IR / data bus

c. PC / control bus

d. PC / address bus

(Trang 72)

Question 210: (Bonus) What are the elements of cache fully associative mapping address?

a. Tag, Set, Byte

b. Tag, Word

c. Tag, Line, Word

d. Tab, Set, Word

Question 211: (Bonus) If the main memory size is 4GB, the cache memory size is 1MB, and the line size is 32 byte, what is the tag size for the 2-way associative cache?

a. 12

b. 13

c. 14

d.  15

Question 212: (Bonus) What is the value of AX after the following code?

MOV AX, 4

MOV BX, 5

DEC BX

ADD AX, BX

INC AX

a. 9

b. 10

c. 11

d. 6

Question 213: (Bonus) What is the output of the following code?

MOV DL, 65

MOV AH, 2

INT 21H

a. ‘a’

b. ‘A’

c. 65

d. Nothing

Question 214: (Bonus) Which is an example of first generation computer?

a. EDVAC

b. EDSAC

c. ENIAC

d. UNIVAC

(Trang 21)

Question 215: (Bonus) Which is the main electronic element used in the third computer generation?

a. Vacuum tube

b. Integrated circuit

c. Transistor

d. Large scale integrated circuit

(Trang 24)

Question 216: (Bonus) PC or Program Counter is used to store the memory address of the _____ instruction

a. current executed

b. executed

c. next executed

d. next

(Trang 65)

Question 217: (Bonus) MBR gets instruction from _____ and forwards it to _____

a. address bus / PC

b. control bus / IR

c. data bus / IR

d. control bus / PC

Question 218: (Bonus) What is the addressing mode of the instruction "LOAD 1000, 500"?

a. Indirect

b. Direct

c. Relative

d. Immediate

Question 219: (Bonus) What are common conditional jump instructions supported by ALU?

a. JE, JZ, JL, JG

b. JE, JZ, JLZ, JG

c. JLE, JGZ, JL, JG

d. JLE, JZ, JLZ, JNG

Question 220: (Bonus) Peripheral devices and a computer communicate through _____

a. ports

b. CPU

c. CU

d. ALU

Question 221: (Bonus) Which is NOT a solution for resource conflicts in CPU pipeline?

a. Improve resource capabilities

b. Memory/Cache should support more read/write operations at a time

c. Split L1 cache into ICache and D-Cache to improve concurrent accesses

d. Delay SUB execution by inserting 3 NO-OP

Question 222: (Bonus) Cache organization is that how cache and _____ are working together

a. CPU

b. CU

c. memory

d. bus system

Question 223: (Bonus) What are the characteristics of the cache direct mapping technique?

a. Simple, slow and flexible mapping

b. Complicated, slow and flexible mapping

c. Simple, fast and fixed mapping

d. Complicated, fast and flexible mapping

(Page 197)

Question 224: (Bonus) What are the base speed of standard CD-ROM and DVD-ROM?

a. 100KB/s and 1350KB/s

b. 100KB/s and 1000KB/s

c. 150KB/s and 1500KB/s

d. 150KB/s and 1350KB/s

(Trang 270-274)

Question 225: (Bonus) What is the RAID level in the following image?

a. RAID 1

b. RAID 0

c. RAID 01

d. RAID 10

(Trang 289)

Question 226: (Bonus) Which is an example of first generation computer?

a. EDVAC

b. EDSAC

c. ENIAC

d. UNIVAC